COMMON COURSE ASSIGNMENT FORMAT

A. Assignment Information

| Particular | Details |
| --- | --- |
| Department: | CSE |
| Programme: | 2ND YEAR |
| Course Code & Course Name | CSA1216 & COMPUTER ARCHITECTURE |
| Academic Year / Batch | 2ND YEAR/ |
| Faculty Name | Dr. Jerin simla |
| Assignment Title | Al Workload Performance Analyzer for Pipeline, Cache and I/O Performance Analysis |
| Date of Issue |  |
| Date of Submission |  |
| Maximum Marks | 100 |
| Course Outcome(s) – CO | CO3, CO4 and CO5 |
| Bloom's Taxonomy Level | L3 – Apply; L4 – Analyze |
| SDG Mapping (SDG 1-17) | SDG 4 – Quality Education; SDG 7 – Affordable and Clean Energy; SDG 9 – Industry, Innovation and Infrastructure |
| Industry / Societal Relevance | The assignment is relevant to AI research data centers and high-performance computing systems. |

B. Assignment Problem / Challenge

Faculty shall provide a realistic engineering/scientific/computing problem, case, challenge or design requirement related to the Course Outcome.

The assignment should preferably require students to:

● Apply course concepts rather than reproduce theory.

● Identify and analyze the problem.

● Consider requirements and constraints.

● Develop or compare alternative solutions where appropriate.

● Use relevant modern engineering/computing/design tools.

● Interpret results.

● Make and justify engineering decisions.

● Consider appropriate technical, economic, environmental, societal, ethical, safety or sustainability factors wherever relevant.

C. Problem Statement

Problem / Case / Design Challenge:

[Faculty to provide the detailed assignment problem here.]

D. Requirements and Constraints

Students shall identify or be provided with relevant requirements and constraints. Examples include:

● Functional requirements

● Performance requirements

● Technical constraints

● Cost, Time

● Resources

● Safety

● Reliability

● Sustainability

● Environmental and Ethical considerations

● Standards/codes

● User requirements

● Manufacturability

● Power/energy

● Security/privacy

Only constraints relevant to the particular course/problem need to be considered.

E. STUDENT WORK

- PROBLEM UNDERSTANDING AND FORMULATION

1.1 Understanding of the Problem

The objective of this assignment is to design and implement an AI Workload Performance Analyzer for an AI Research Data Center. The system analyzes the performance of AI workloads by considering three major components of computer architecture: processor pipeline performance, cache memory performance, and I/O performance.

AI workloads such as image processing and machine learning applications require large amounts of computation, memory access, and data transfer. Therefore, processor pipeline efficiency, cache performance, and I/O efficiency have a significant impact on overall system performance.

The proposed analyzer accepts information about an AI workload, including Job ID, number of images, image size, and processor instruction sequences. It simulates the execution of instructions through a five-stage pipeline consisting of Instruction Fetch (IF), Instruction Decode (ID), Execute (EX), Memory Access (MEM), and Write Back (WB).

The system identifies different types of pipeline hazards. Data hazards are identified by analyzing source and destination register dependencies. Control hazards are identified using branch instructions. Structural hazards are identified by checking conflicts in processor resources.

The analyzer also examines memory accesses generated by LOAD and STORE instructions to determine cache hits, cache misses, main-memory accesses, hit/miss ratios, and Average Memory Access Time (AMAT).

Finally, the system evaluates image-data transfer using interrupt-driven I/O and Direct Memory Access (DMA). The two methods are compared based on transfer time, CPU intervention, and CPU utilization.

1.2 Expected Outcomes

The expected outcomes of the project are:

- To simulate the execution of instructions using a five-stage processor pipeline.

- To identify data, control, and structural hazards.

- To determine affected instructions and pipeline stall or flush cycles.

- To calculate total pipeline execution cycles.

- To calculate CPI and execution time.

- To calculate pipeline speed-up.

- To analyze L1, L2, and L3 cache performance.

- To determine cache hits and misses.

- To calculate cache hit and miss ratios.

- To determine the number of main-memory accesses.

- To calculate Average Memory Access Time (AMAT).

- To calculate image-data transfer requirements.

- To compare interrupt-driven I/O and DMA.

- To determine CPU intervention and CPU utilization.

- To identify the major performance bottleneck.

- To generate a final performance report with suitable recommendations.

1.3 Input Data

The system requires the following input data:

• Job ID
• Number of images
• Image size
• Instruction ID
• Operation type
• Source register or registers
• Destination register
• Memory address for LOAD and STORE instructions
• Cache size
• Cache block size
• Cache access latency
• Storage transfer rate
• I/O transfer parameters

The instruction sequence may contain operations such as LOAD, STORE, arithmetic operations, and BRANCH instructions.

1.4 Assumptions

The following assumptions are considered for the analysis:

• The processor uses a five-stage pipeline consisting of IF, ID, EX, MEM, and WB.
• Instructions are issued in program order.
• Data hazards are identified using source and destination register dependencies.
• Branch instructions are considered potential control hazards.
• Structural hazards occur when two instructions require the same processor resource at the same time.
• Cache memory is organized into blocks.
• LOAD and STORE instructions generate memory accesses.
• Cache analysis considers L1, L2, and L3 caches followed by main memory.
• DMA transfers data directly between storage and main memory with minimum CPU intervention.
• Transfer time depends on the total amount of data and storage transfer rate.
• The project uses a simplified architectural model for educational analysis.

1.5 Constraints

The major constraints of the system are:

• Input instruction data must follow the specified format.
• Register names must be valid.
• Memory addresses must be valid numerical values.
• Cache sizes and block sizes must be properly specified.
• Invalid or incomplete input must be handled using exception handling.
• The system should support different workload sizes.
• The implementation should use Python classes, constructors, inheritance, polymorphism, exception handling, file handling, and suitable data structures.

- APPLICATION OF COURSE KNOWLEDGE

The assignment applies important concepts from Computer Architecture to evaluate AI workload performance.

2.1 Five-Stage Pipeline

A processor pipeline divides instruction execution into multiple stages. The five stages considered in this project are:

IF – Instruction Fetch:
The instruction is fetched from memory.

ID – Instruction Decode:
The instruction is decoded and the required registers and resources are identified.

EX – Execute:
The arithmetic, logical, or branch operation is performed.

MEM – Memory Access:
Memory read or write operations are performed for LOAD and STORE instructions.

WB – Write Back:
The calculated result is written into the destination register.

For N instructions, the ideal number of cycles for a five-stage pipeline is:

Ideal Pipeline Cycles = N + (Pipeline Stages - 1)

Since the processor has five stages:

Ideal Pipeline Cycles = N + 4

2.2 Data Hazards

A data hazard occurs when an instruction depends on the result produced by a previous instruction.

For example:

I1: ADD R1, R2 → R3
I2: SUB R3, R4 → R5

Instruction I2 requires register R3, which is produced by instruction I1. Therefore, a dependency exists between the two instructions.

The analyzer identifies a data hazard by comparing:

Destination Register of Previous Instruction
with
Source Register of Current Instruction

When a dependency is detected, one or more stall cycles may be introduced depending on the pipeline model.

2.3 Control Hazards

Control hazards occur because of branch instructions.

For example:

I5: BRANCH R1, TARGET

The processor may fetch subsequent instructions before determining the result of the branch. If the branch changes the execution path, the incorrectly fetched instructions may have to be flushed.

The analyzer identifies BRANCH instructions and calculates the corresponding control-hazard penalty.

2.4 Structural Hazards

Structural hazards occur when multiple instructions require the same hardware resource at the same time.

For example, if instruction execution and memory access require a shared hardware resource simultaneously, a resource conflict may occur.

The analyzer checks processor-resource usage during different pipeline stages and identifies conflicts that require additional stall cycles.

2.5 Pipeline Performance Metrics

CPI:

CPI is calculated using:

CPI = Total Pipeline Cycles / Number of Instructions

For example, if the number of instructions is 8 and the total pipeline cycles are 18:

CPI = 18 / 8

CPI = 2.25

Execution Time:

Execution Time = Total Cycles × Clock Cycle Time

Assuming a processor frequency of 2 GHz:

Clock Cycle Time = 1 / 2 GHz

Clock Cycle Time = 0.5 ns

Therefore:

Execution Time = 18 × 0.5

Execution Time = 9 ns

Pipeline Speed-up:

For a five-stage non-pipelined processor:

Non-Pipelined Cycles = 5 × N

For 8 instructions:

Non-Pipelined Cycles = 5 × 8

Non-Pipelined Cycles = 40 cycles

Therefore:

Speed-up = Non-Pipelined Cycles / Pipelined Cycles

Speed-up = 40 / 18

Speed-up = 2.22

Thus, the example workload achieves approximately 2.22 times speed-up using the pipeline.

- CACHE MEMORY ANALYSIS

Cache memory is a small and fast memory located close to the processor. It reduces the average time required to access frequently used data.

The system analyzes three levels of cache:

• L1 Cache
• L2 Cache
• L3 Cache

3.1 Cache Parameters

Example cache configuration:

L1 Cache Size = 128 Bytes
L2 Cache Size = 256 Bytes
L3 Cache Size = 512 Bytes
Block Size = 64 Bytes
L1 Access Latency = 1 ns
L2 Access Latency = 5 ns
L3 Access Latency = 20 ns
Main Memory Latency = 100 ns

Number of Cache Blocks:

Number of Cache Blocks = Cache Size / Block Size

For L1:

Number of L1 Blocks = 128 / 64

Number of L1 Blocks = 2

For L2:

Number of L2 Blocks = 256 / 64

Number of L2 Blocks = 4

For L3:

Number of L3 Blocks = 512 / 64

Number of L3 Blocks = 8

3.2 Cache Hit and Cache Miss

A cache hit occurs when the required memory block is already present in the cache.

A cache miss occurs when the required block is not present in the cache and must be obtained from the next memory level.

The analyzer processes memory addresses from LOAD and STORE instructions and converts them into cache block numbers.

Block Number = Memory Address / Block Size

For example:

Memory Address = 128 Bytes

Block Size = 64 Bytes

Block Number = 128 / 64

Block Number = 2

3.3 Cache Hit Ratio

The cache hit ratio is calculated using:

Hit Ratio = Number of Hits / Total Number of Accesses

The miss ratio is:

Miss Ratio = Number of Misses / Total Number of Accesses

or

Miss Ratio = 1 - Hit Ratio

These metrics are calculated for the different cache levels.

3.4 Average Memory Access Time (AMAT)

Average Memory Access Time is used to measure the average time required to access data from the memory hierarchy.

The general formula is:

AMAT = Hit Time + (Miss Rate × Miss Penalty)

For a multi-level cache:

AMAT = L1 Hit Time + L1 Miss Rate × (L2 Hit Time + L2 Miss Rate × (L3 Hit Time + L3 Miss Rate × Main Memory Penalty))

A lower AMAT indicates better memory-system performance.

- I/O AND DMA ANALYSIS

AI workloads commonly process large image datasets. Loading these images from storage to memory can become a major performance bottleneck.

For example, consider:

Number of Images = 100
Image Size = 1 MB
Storage Transfer Rate = 100 MB/s

4.1 Total Data Size

Total Data Size = Number of Images × Image Size

Total Data Size = 100 × 1 MB

Total Data Size = 100 MB

4.2 Transfer Time

Transfer Time = Total Data Size / Transfer Rate

Transfer Time = 100 MB / 100 MB/s

Transfer Time = 1 second

4.3 Interrupt-Driven I/O

In interrupt-driven I/O, the CPU is notified whenever a transfer requires attention.

If every image generates one transfer request:

Number of CPU Interventions = 100

Assuming the CPU spends 1 ms servicing each request:

Total CPU Service Time = Number of I/O Requests × CPU Service Time per Request

Total CPU Service Time = 100 × 1 ms

Total CPU Service Time = 100 ms

CPU Utilization:

CPU Utilization = (Total CPU Service Time / Total Transfer Time) × 100

CPU Utilization = (100 ms / 1000 ms) × 100

CPU Utilization = 10%

4.4 Direct Memory Access (DMA)

Direct Memory Access allows a DMA controller to transfer a large block of data directly between the storage device and memory without requiring continuous CPU intervention.

For the same 100-image workload, the data can be transferred using a DMA operation.

Assuming one DMA setup requiring 1 ms of CPU time:

CPU Interventions = 1

CPU Service Time = 1 ms

DMA CPU Utilization:

DMA CPU Utilization = (DMA CPU Service Time / Total Transfer Time) × 100

DMA CPU Utilization = (1 ms / 1000 ms) × 100

DMA CPU Utilization = 0.1%

Therefore, DMA significantly reduces CPU involvement compared with interrupt-driven I/O.

- SOLUTION / DESIGN / METHODOLOGY

The proposed system is divided into three major modules:

- Pipeline Analysis Module

- Cache Analysis Module

- I/O and DMA Analysis Module

The overall workflow is:

AI Workload Input
↓
Input/Data Manager
↓
Pipeline Analysis
↓
Cache Analysis
↓
I/O and DMA Analysis
↓
Performance Analyzer
↓
Final Performance Report

5.1 Pipeline Analysis Module

The pipeline module performs the following steps:

- Read the instruction sequence.

- Assign instructions to pipeline stages.

- Identify source and destination registers.

- Detect data dependencies.

- Identify data hazards.

- Detect branch instructions.

- Identify control hazards.

- Check processor-resource conflicts.

- Calculate stall and flush cycles.

- Calculate total pipeline cycles.

- Calculate CPI.

- Calculate execution time.

- Calculate pipeline speed-up.

5.2 Cache Analysis Module

The cache module performs the following steps:

- Extract memory addresses from LOAD and STORE instructions.

- Convert memory addresses into cache block numbers.

- Check L1 cache.

- If L1 misses, check L2 cache.

- If L2 misses, check L3 cache.

- If L3 misses, access main memory.

- Record cache hits and misses.

- Calculate hit ratios and miss ratios.

- Calculate main-memory accesses.

- Calculate AMAT.

5.3 I/O Analysis Module

The I/O module performs the following steps:

- Read the number of images.

- Read the image size.

- Calculate total data size.

- Read the storage transfer rate.

- Calculate transfer time.

- Model interrupt-driven I/O.

- Model DMA.

- Calculate CPU interventions.

- Calculate CPU utilization.

- Compare interrupt-driven I/O and DMA.

- OBJECT-ORIENTED DESIGN

The system is designed using object-oriented programming concepts.

The major classes are:

• Instruction
• PipelineAnalyzer
• CacheLevel
• CacheAnalyzer
• IOAnalyzer
• PerformanceReport

6.1 Constructors

Constructors are used to initialize objects with required values such as instruction information, cache parameters, and workload parameters.

6.2 Inheritance

Inheritance can be used to create specialized analyzer classes from a common analyzer class.

For example:

Analyzer
|
|---- PipelineAnalyzer
|
|---- CacheAnalyzer
|
|---- IOAnalyzer

6.3 Polymorphism

Polymorphism allows different analyzer classes to implement a common analysis operation while performing different types of processing.

This improves modularity and makes the system easier to extend.

- USE OF MODERN TOOLS

Python is used as the primary implementation language because it provides strong support for object-oriented programming, data structures, file handling, exception handling, and mathematical calculations.

The following concepts and tools are used:

• Python programming language
• Object-oriented programming
• Classes and objects
• Constructors
• Inheritance
• Polymorphism
• Lists
• Dictionaries
• File handling
• Exception handling
• Mathematical calculations
• Performance metrics
• Tabular result generation

Input data can be stored in a text file or CSV file. This allows different AI workloads to be tested without modifying the main program.

- RESULTS AND VALIDATION

For an example workload containing 8 instructions, the ideal five-stage pipeline requires:

Ideal Pipeline Cycles = N + 4

Ideal Pipeline Cycles = 8 + 4

Ideal Pipeline Cycles = 12 cycles

Assume that the analysis identifies:

Data-Hazard Stalls = 3 cycles
Control-Hazard Flushes = 2 cycles
Structural-Hazard Stalls = 1 cycle

Therefore:

Total Pipeline Cycles = Ideal Pipeline Cycles + Data-Hazard Stalls + Control-Hazard Flushes + Structural-Hazard Stalls

Total Pipeline Cycles = 12 + 3 + 2 + 1

Total Pipeline Cycles = 18 cycles

8.1 Pipeline Results

Number of Instructions = 8
Ideal Pipeline Cycles = 12
Data Hazard Stalls = 3
Control Hazard Flushes = 2
Structural Hazard Stalls = 1
Total Pipeline Cycles = 18
CPI = 2.25
Clock Frequency = 2 GHz
Clock Cycle Time = 0.5 ns
Execution Time = 9 ns
Pipeline Speed-up = 2.22×

8.2 Cache Results

The cache analyzer records the number of hits and misses at each cache level.

L1 cache has high miss activity for the sample memory-access pattern.

L2 cache can serve repeated memory blocks that are not available in L1.

L3 cache can serve some blocks that miss in both L1 and L2.

Main memory is accessed when the required block is not available in any cache level.

The hit ratio, miss ratio, main-memory accesses, and AMAT are calculated from the memory-address sequence.

8.3 I/O Results

Total Data = 100 MB

Transfer Time = 1 second

Interrupt-Driven I/O:

CPU Interventions = 100
CPU Service Time = 100 ms
CPU Utilization = 10%

DMA:

CPU Interventions = 1
CPU Service Time = 1 ms
CPU Utilization = 0.1%

The results show that DMA significantly reduces CPU intervention and CPU utilization compared with interrupt-driven I/O.

- ANALYSIS AND ENGINEERING DECISION

The analysis shows that system performance depends on the interaction between processor execution, memory hierarchy, and I/O operations.

Pipeline hazards increase the number of execution cycles and therefore increase CPI. Data hazards can be reduced using techniques such as forwarding and instruction scheduling. Control hazards can be reduced using branch prediction and early branch resolution. Structural hazards can be reduced by providing sufficient processor resources.

Cache performance also has a major effect on memory access time. A high cache hit ratio reduces the number of accesses to slower memory levels. Therefore, suitable cache organization and improved data locality can improve system performance.

For large AI image datasets, I/O can become a significant bottleneck. Interrupt-driven I/O requires repeated CPU intervention for individual transfer requests. DMA is more efficient because it allows large blocks of data to be transferred with minimal CPU involvement.

Engineering Decision:

Based on the analysis, DMA is preferred for large AI image-data transfers because it significantly reduces CPU intervention and CPU utilization.

The following improvements are recommended:

- Use DMA for large data transfers.

- Improve cache locality in AI workloads.

- Use suitable cache organization.

- Reduce unnecessary data dependencies.

- Use forwarding or instruction scheduling to reduce pipeline stalls.

- Use branch prediction to reduce control-hazard penalties.

- Provide adequate processor resources to reduce structural hazards.

- BROADER CONSIDERATIONS

10.1 Sustainability

Efficient processor, cache, and I/O utilization can reduce unnecessary computation and energy consumption. DMA reduces CPU workload and allows the processor to spend less time handling data-transfer operations.

10.2 Environmental Impact

AI data centers consume significant amounts of electrical energy. Improving processor utilization, cache efficiency, and data-transfer efficiency can contribute to lower energy consumption and reduced environmental impact.

10.3 Economic Considerations

Improved system performance can reduce execution time and increase data-center throughput. Efficient resource utilization can reduce operational costs and improve the efficiency of computing infrastructure.

10.4 Social Relevance

AI systems are increasingly used in education, healthcare, scientific research, and other applications. Efficient computing infrastructure allows AI workloads to be processed faster and more efficiently.

10.5 Professional Responsibility

The system should produce accurate and reproducible performance measurements. The assumptions used in the simulation must be clearly documented, and the limitations of the simplified architectural model must be acknowledged.

- CONCLUSION

The AI Workload Performance Analyzer provides a systematic method for evaluating the performance of AI workloads using important Computer Architecture concepts.

The project analyzes five-stage processor pipelines, data hazards, control hazards, structural hazards, cache performance, AMAT, I/O operations, and DMA. The results demonstrate that pipeline hazards can increase execution cycles, cache misses can increase memory access time, and inefficient I/O can increase CPU overhead.

For large-scale AI workloads involving image datasets, DMA provides a significant advantage because it minimizes CPU intervention during data transfer. Similarly, improving cache locality and reducing pipeline hazards can improve overall processor performance.

Therefore, the combined analysis of pipeline, cache, and I/O performance provides a complete understanding of the system and helps identify major performance bottlenecks and appropriate engineering solutions.

- STUDENT REFLECTION

Through this assignment, I gained a better understanding of how different components of Computer Architecture affect the performance of real-world applications.

I learned how instructions move through the five-stage processor pipeline and how data, control, and structural hazards can introduce additional execution cycles.

I also understood the importance of cache memory in reducing memory access time. The analysis of L1, L2, and L3 caches helped me understand how cache hits and misses influence AMAT and overall system performance.

The I/O and DMA analysis helped me understand why continuous CPU involvement in large data transfers can be inefficient. Comparing interrupt-driven I/O with DMA showed how DMA can reduce CPU utilization and improve system efficiency.

Overall, this assignment helped me connect theoretical concepts from Computer Architecture with practical performance analysis. It improved my understanding of how processor, memory, and I/O components work together in modern computing systems.

F. Common Assessment Rubric

| Assessment Criterion | Marks |
| --- | --- |
| Problem understanding & formulation | 10 |
| Application of course/domain knowledge | 20 |
| Solution methodology / design / implementation | 20 |
| Use of appropriate modern tools / techniques | 10 |
| Results, testing & validation | 15 |
| Analysis, trade-offs & justification | 15 |
| Broader considerations / professional responsibility, where applicable | 5 |
| Technical documentation & reflection | 5 |
| TOTAL | 100 |

G. CO–PO–Assessment Mapping

| Assessment Component |  | CO PO(s) | Bloom's Level | Marks |
| --- | --- | --- | --- | --- |
| Problem formulation |  |  | L3/L4 | 10 |
| Application of knowledge |  |  | L3/L4 | 20 |
| Solution / Design / Implementation |  |  | L4/L5/L6 | 20 |
| Modern tool usage |  |  | L3/L4 | 10 |
| Validation |  |  | L4/L5 | 15 |
| Analysis & justification |  |  | L4/L5 | 15 |
| Broader considerations |  |  | L4/L5 | 5 |
| Documentation & reflection |  |  | L3/L4 | 5 |

Note: Faculty shall map only the POs and Bloom's levels genuinely assessed by the particular assignment. Every assignment is not required to map to every PO.

H. Faculty Evaluation & Continuous Improvement

CO Attainment

| Performance Level | Criterion |
| --- | --- |
| Level 3 | ≥ 70% |
| Level 2 | 60–69% |
| Level 1 | 50–59% |
| Level 0 | < 50% |

Target CO Attainment: __________

Actual CO Attainment: __________

Faculty Analysis

Areas in which students performed well:

Areas requiring improvement:

Corrective / Improvement Action:

Action to be implemented in the next offering:

Documents to be Maintained

The department/course faculty shall maintain:

1. Assignment question/problem statement

2. CO–PO and Bloom's mapping

3. Assessment rubric

4. Student submissions

5. Tool/simulation/experimental evidence, where applicable 6. Evaluated scripts/reports

7. Marks and rubric-wise assessment

8. CO attainment analysis

9. Sample student work representing different performance levels 10. Faculty analysis and continuous-improvement action