SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 3- DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL2- DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | N.Nesarajan | Reg.No.: | 192511386 |
| Department: | CSE | Date: | 12-08-2026 |

ANNEXURE A

- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

- A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry look-ahead adder, and explain how it reduces delay.

- A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-pair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

- An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

ANSWERS

Q1. Signed Integer Addition using 2’s Complement

Problem:
A program performs signed integer addition using 2’s complement, but produces an incorrect result when adding large values such as +120 and +50 using 8-bit representation.

Step 1: Convert the numbers to binary

For +120:

120 = 01111000

For +50:

50 = 00110010

Step 2: Perform binary addition

01111000   (+120)

+ 00110010   (+50)

------------

10101010

The resulting binary value is:

10101010

Step 3: Identify the problem

In an 8-bit signed 2's complement system:

- Positive numbers: 00000000 to 01111111

- Negative numbers: 10000000 to 11111111

- Maximum positive value = +127

- Minimum negative value = −128

The mathematical result should be:

120 + 50 = 170

But +170 cannot be represented using 8-bit signed 2's complement.

The result 10101010 has its MSB as 1, so it is interpreted as a negative number.

Therefore, signed integer overflow has occurred.

Overflow Detection

Overflow occurs when:

- Two positive numbers produce a negative result, or

- Two negative numbers produce a positive result.

Here:

Positive + Positive = Negative

↓

Overflow

Debugging Solution

The program should check the sign bits before accepting the result.

For addition:

if (sign(A) == sign(B)) {

if (sign(Result) != sign(A))

Overflow = 1;

}

Correct Handling

The program should:

- Perform the addition.

- Check for overflow.

- Set an overflow flag if the result is outside the representable range.

- Report that the result cannot be represented in 8-bit signed format.

Conclusion

The error is not in the binary addition itself. The problem is 8-bit signed overflow. Increasing the number of bits, such as using 16-bit integers, or detecting overflow before accepting the result solves the problem.

Q2. Ripple Carry Adder vs Carry Look-Ahead Adder

The problem states that a ripple carry adder causes significant delay because the carry must propagate through each bit.

Ripple Carry Adder

In a ripple carry adder, each full adder waits for the carry from the previous stage.

For a 4-bit addition:

A0 B0 → FA0 → C1

↓

A1 B1 → FA1 → C2

↓

A2 B2 → FA2 → C3

↓

A3 B3 → FA3 → C4

The carry moves from:

Bit 0 → Bit 1 → Bit 2 → Bit 3

This creates a delay.

Example

Suppose:

1111

+ 0001

------

10000

The carry must travel through several stages before the final result is known.

Problem

For an n-bit ripple carry adder, the delay increases approximately with the number of bits.

Therefore:

More bits → More carry propagation → More delay

Optimized Solution: Carry Look-Ahead Adder

A Carry Look-Ahead Adder (CLA) calculates carries in advance rather than waiting for them to ripple through every stage.

It uses:

Generate:

Gi = Ai · Bi

Propagate:

Pi = Ai ⊕ Bi

Carry equation:

Ci+1 = Gi + PiCi

For example:

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

The carries can therefore be calculated much faster.

Advantages

| Ripple Carry Adder | Carry Look-Ahead Adder |
| --- | --- |
| Carry moves sequentially | Carry calculated in advance |
| Higher delay | Lower delay |
| Simple design | More complex design |
| Suitable for smaller systems | Suitable for high-speed processors |

Conclusion

The bottleneck in the ripple carry adder is carry propagation. Replacing it with a Carry Look-Ahead Adder reduces the arithmetic delay and improves processor performance. The assignment specifically asks for CLA as the optimized solution.

Q3. Booth's Algorithm – Signed Multiplication

The problem involves a multiplication unit using Booth's algorithm that produces incorrect results for some negative operands. The debugging must focus on bit-pair checking, arithmetic shifts, and sign extension.

Booth's Algorithm

Booth's algorithm is used for signed binary multiplication using 2's complement.

It uses:

- Multiplicand: M

- Multiplier: Q

- Accumulator: A

- Extra bit: Q−1

Initially:

A = 0

Q = Multiplier

Q−1 = 0

M = Multiplicand

Bit-Pair Checking

The pair Q0 Q−1 is examined.

| Q0 Q−1 | Operation |
| --- | --- |
| 00 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |
| 11 | No operation |

After the operation, perform an arithmetic right shift on:

A, Q, Q−1

The process is repeated for the number of bits.

Possible Errors

1. Incorrect bit-pair checking

The program may incorrectly check:

Q1 Q0

instead of:

Q0 Q−1

This produces an incorrect multiplication result.

2. Logical shift instead of arithmetic shift

For signed numbers, an arithmetic right shift must be used.

An arithmetic shift preserves the sign bit.

Example:

10110010

Arithmetic right shift:

11011001

The leading 1 is preserved.

A logical shift would incorrectly insert 0.

3. Incorrect sign extension

When negative numbers are used, the sign bit must be extended when increasing the number of bits.

For example:

8-bit:   11110110

Sign-extended to 16 bits:

11111111 11110110

The leading 1s must be preserved.

Debugging Solution

The implementation should:

- Initialize Q−1 = 0.

- Check Q0 Q−1.

- Perform addition/subtraction correctly.

- Use arithmetic right shift.

- Preserve the sign bit.

- Repeat for the correct number of iterations.

- Combine A and Q to obtain the final product.

Conclusion

The most likely causes of incorrect negative multiplication are wrong bit-pair checking, logical shifting, or incorrect sign extension. Correct implementation of these operations makes Booth's algorithm work properly for signed operands.

Q4. Restoring Division vs Non-Restoring Division

The embedded system uses the restoring division algorithm, but repeated restoration steps increase execution time.

Restoring Division

In restoring division, the following steps are performed repeatedly:

- Shift the partial remainder.

- Subtract the divisor.

- Check the sign of the remainder.

- If the result is negative, restore the previous remainder.

- Set the quotient bit.

The restoration operation is an extra step.

Example Workflow

Shift

↓

Subtract divisor

↓

Is remainder negative?

↓

Yes

↓

Restore remainder

↓

Continue

The repeated restore operation increases execution time.

Problem

The main inefficiency is:

Subtract → Negative result → Restore

This can happen repeatedly during division.

Therefore:

More restoration operations

↓

More clock cycles

↓

Higher execution time

Optimized Solution: Non-Restoring Division

Non-restoring division avoids immediately restoring the remainder.

Instead:

- If the current remainder is positive, subtract the divisor.

- If the current remainder is negative, add the divisor.

Basic Algorithm

If R ≥ 0:

R = 2R - M

Else:

R = 2R + M

The quotient bit is determined based on the sign of the resulting remainder.

At the end, if the remainder is negative, a final correction is performed.

Advantages

| Restoring Division | Non-Restoring Division |
| --- | --- |
| Restores remainder repeatedly | Avoids repeated restoration |
| More operations | Fewer operations |
| Higher execution time | Lower execution time |
| Simpler concept | More efficient |

Conclusion

The main bottleneck in restoring division is the repeated restoration operation. Non-restoring division removes these unnecessary intermediate restoration steps, improving the performance of the embedded processor.

Q5. IEEE 754 Floating-Point Addition

The scientific application produces inaccurate results when adding very small and very large floating-point numbers. The problem requires analysis of exponent alignment, normalization, and rounding.

IEEE 754 Single Precision

A single-precision floating-point number contains:

1 Sign bit

8 Exponent bits

23 Fraction bits

Total:

32 bits

Format:

| Sign | Exponent | Fraction |

|  1   |    8     |    23    |

Step 1: Compare Exponents

Before adding two floating-point numbers, their exponents must be equal.

Suppose:

A = 1.234 × 10^8

B = 1.234 × 10^2

The exponents are different.

The smaller number must be shifted to align the exponents.

A = 1.234 × 10^8

B = 0.000001234 × 10^8

Step 2: Add the Mantissas

After exponent alignment, the mantissas are added.

However, if one number is extremely small compared with the other, its significant bits may be lost.

For example:

Large number + Very small number

The small number may have little or no effect on the final stored result.

Step 3: Normalization

After addition, the result must be normalized.

The mantissa should generally have the form:

1.xxxxx × 2^n

If necessary:

- Shift the mantissa.

- Adjust the exponent.

Step 4: Rounding

Single precision has only 23 fraction bits.

If more bits are produced during calculation, they must be rounded.

This can introduce a small error.

Main Problems

The inaccurate result can be caused by:

- Exponent alignment

- Loss of significant bits

- Normalization

- Rounding

- Limited precision of single precision

Optimization Techniques

1. Use Double Precision

Double precision provides more fraction bits than single precision.

Therefore, it can represent numbers with greater accuracy.

Single Precision → 32 bits

Double Precision → 64 bits

For scientific applications requiring high accuracy, double precision is often preferable.

2. Use Appropriate Algorithms

The program can reduce numerical errors by:

- Ordering additions carefully.

- Avoiding unnecessary subtraction of nearly equal numbers.

- Using numerically stable algorithms.

- Using higher precision for intermediate calculations.

3. Preserve Guard Digits

Extra intermediate bits can be retained before rounding.

This helps reduce precision loss.

Conclusion

The main reason for inaccurate results is the limited precision of IEEE 754 single precision, especially when numbers have very different magnitudes. Using double precision and numerically stable algorithms can significantly improve accuracy. The assignment specifically identifies exponent alignment, normalization, rounding, and precision as areas to debug.

Code of Conduct:

I N.Nesarajan/192511386 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Identification of Errors / Bugs (20%) | Debugging Approach & Analysis (20%) | Correctness of Debugged Solution (25%) | Optimization Techniques Applied (20%) | Testing and Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |