SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- EXPERIMENTS

| Course Code: | CSA1205 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 1- EXPERIMENTS |
| Weightage: | 40% | Total Marks: | 25 |
| CO3 Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6) |  |  |
| Student Name: | N.Nesarajan | Reg.No.: | 192511386 |
| Department: | CSE | Date: | 14-08-2026 |

ANNEXURE A

- Design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR. Use multiple registers to demonstrate how data is transferred between them during execution, and analyze the sequence of operations and the role of the ALU in processing data.

- Create a model of a computer system using single-bus and multi-bus organization, and perform data transfer operations between registers, memory, and I/O. Compare the time taken and efficiency of both approaches, and analyze how multiple bus structures improve parallel data transfer and reduce bottlenecks.

- Simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back) and execute a set of instructions. Measure the performance in terms of speedup and throughput, and compare it with a non-pipelined system to evaluate the effectiveness of pipelining.

- Develop a pipeline execution scenario where data hazards, control hazards, and structural hazards occur. Observe their impact on instruction execution, and implement techniques such as stalling, data forwarding, and branch prediction to resolve these hazards and improve performance.

- Analyze a processor supporting superscalar execution, including out-of-order and speculative execution, and extend the study to include hyper-threading and simultaneous multithreading (SMT). Evaluate how multiple instructions are executed in parallel and compare the performance improvements in terms of instruction throughput and resource utilization.

Code of Conduct:

I N.Nesarajan/192511386 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Understanding of Concepts (20%) | Experimental Design (20%) | Use of Tools (25%) | Analysis of Results (20%) | Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

Answers

1. Aim

To design and simulate a simple processor datapath that performs register transfer operations and basic arithmetic and logic operations such as addition, subtraction, AND, and OR using multiple registers and an Arithmetic Logic Unit (ALU).

2. Objectives

- To understand the concept of register transfer operations.

- To demonstrate the transfer of data between multiple registers.

- To perform arithmetic operations using the ALU.

- To perform logical AND and OR operations using the ALU.

- To analyze the sequence of operations during instruction execution.

- To understand the role of the ALU in processing data.

3. Theory

A register is a small and fast storage location inside a processor used to temporarily store data, instructions, or intermediate results. Register Transfer Language (RTL) is used to represent the movement of data between registers. For example, the operation R2 ← R1 means that the contents of register R1 are transferred to register R2.

The Arithmetic Logic Unit (ALU) is a major component of the processor datapath. It performs arithmetic operations such as addition and subtraction and logical operations such as AND and OR. The ALU receives operands from registers, performs the selected operation, and stores the result in a destination register.

In this experiment, multiple registers such as R1, R2, R3, and R4 are used. Data is first loaded into registers and then transferred between them. The ALU performs the required operation on the register contents. The result is finally stored in another register for further processing.

4. Register Transfer Operations

Register transfer operations describe how data moves from one register to another.

For example:

- R1 ← R2 : Contents of R2 are transferred to R1.

- R2 ← R3 : Contents of R3 are transferred to R2.

- R3 ← R1 : Contents of R1 are transferred to R3.

Consider:

R1 = 10

R2 = 20

The operation:

R3 ← R1

results in:

R3 = 10

The original contents of R1 remain unchanged.

5. ALU Operations

The ALU performs four basic operations in this experiment.

Addition

The ALU adds the contents of two registers.

R3 ← R1 + R2

If:

R1 = 10

R2 = 5

then:

R3 = 15

Subtraction

The ALU subtracts the second operand from the first operand.

R3 ← R1 - R2

If:

R1 = 10

R2 = 5

then:

R3 = 5

AND Operation

The ALU performs a bit-by-bit logical AND operation.

R3 ← R1 AND R2

For example:

R1 = 1010

R2 = 1100

Result:

R3 = 1000

OR Operation

The ALU performs a bit-by-bit logical OR operation.

R3 ← R1 OR R2

For example:

R1 = 1010

R2 = 1100

Result:

R3 = 1110

6. Proposed Datapath

The basic datapath consists of:

- Register R1

- Register R2

- Register R3

- Register R4

- ALU

- Control unit

- Data buses

The registers provide input data to the ALU. The control unit selects the required ALU operation. The ALU processes the data and sends the result to the destination register.

Block Representation

+---------+

R1 --------->|         |

|         |

R2 --------->|   ALU   |---------> R3

|         |

|         |

+---------+

|

Control Unit

|

Select ADD / SUB /

AND / OR

7. Algorithm

- Start the simulation.

- Initialize the registers.

- Load the first operand into R1.

- Load the second operand into R2.

- Transfer data between registers when required.

- Send the contents of R1 and R2 to the ALU.

- Select the addition operation.

- Store the result in R3.

- Select the subtraction operation.

- Store the result in R3.

- Select the AND operation.

- Store the result in R3.

- Select the OR operation.

- Store the result in R3.

- Display the register contents and ALU results.

- Stop the simulation.

8. RTL Sequence

The following RTL sequence demonstrates the execution:

R1 ← 10

R2 ← 5

R3 ← R1 + R2

R4 ← R1 - R2

R3 ← R1 AND R2

R4 ← R1 OR R2

For a simple register-transfer demonstration:

R1 ← R2

R2 ← R3

R3 ← R1

These operations demonstrate how information is transferred between registers during processor execution.

9. Sample Simulation

Assume:

R1 = 10

R2 = 5

Addition

Operation: R3 ← R1 + R2

R1 = 10

R2 = 5

R3 = 15

Subtraction

Operation: R3 ← R1 - R2

R1 = 10

R2 = 5

R3 = 5

AND

Using 4-bit binary representation:

R1 = 1010

R2 = 0101

1010

AND  0101

---------

0000

Therefore:

R3 = 0000

OR

R1 = 1010

R2 = 0101

1010

OR   0101

---------

1111

Therefore:

R3 = 1111

10. Sample Output

REGISTER TRANSFER AND ALU SIMULATION

Initial Register Values:

R1 = 10

R2 = 5

R3 = 0

R4 = 0

Register Transfer:

R3 <- R1

R3 = 10

Addition:

R3 <- R1 + R2

R3 = 15

Subtraction:

R4 <- R1 - R2

R4 = 5

AND Operation:

R3 <- R1 AND R2

R3 = 0

OR Operation:

R4 <- R1 OR R2

R4 = 15

Simulation Completed Successfully.

11. Role of ALU

The ALU is responsible for performing the actual arithmetic and logical processing. It receives operands from the registers and performs the operation selected by the control unit. For addition and subtraction, the ALU performs arithmetic calculations. For AND and OR, it performs bitwise logical operations. The output generated by the ALU is transferred to the required destination register.

Therefore, the ALU acts as the main computational unit of the datapath.

12. Analysis of Results

The simulation demonstrates that data can be transferred between registers and processed by the ALU. The register-transfer operations show the movement of data within the processor datapath. The addition and subtraction operations produce arithmetic results, while AND and OR produce bitwise logical results. The experiment also shows that the control unit determines which ALU operation is performed. Using separate registers for operands and results makes the execution sequence easy to observe and understand.

13. Result

The register transfer and ALU system was successfully designed and simulated. Data was transferred between multiple registers, and the ALU successfully performed addition, subtraction, AND, and OR operations. The experiment demonstrates the basic operation of a processor datapath and the important role played by registers and the ALU during data processing.

14. Conclusion

This experiment provides an understanding of how data moves between processor registers and how the ALU performs arithmetic and logical operations. Register transfer operations demonstrate internal data movement, while the ALU performs the required computation. The experiment therefore gives a basic understanding of processor organization and datapath operation.

Q2.

1. Aim

To design and simulate a computer system using single-bus and multi-bus organization, perform data transfer operations between registers, memory, and I/O devices, and compare their execution time, efficiency, parallelism, and bottlenecks.

2. Objectives

- To understand single-bus organization.

- To understand multi-bus organization.

- To perform data transfer between registers, memory, and I/O.

- To compare the execution time of single-bus and multi-bus systems.

- To analyze how multiple buses improve parallel data transfer.

- To understand how multi-bus organization reduces data-transfer bottlenecks.

3. Theory

A bus is a communication path used to transfer data between different components of a computer system. Registers, memory, ALU, and I/O devices communicate with each other through buses.

In a single-bus organization, all components share one common bus. Only one major data transfer can use the bus at a time. This makes the design simple and less expensive, but it can create a bottleneck when several components need to transfer data simultaneously.

In a multi-bus organization, multiple buses are available for data transfer. Different transfers can take place simultaneously, depending on the processor design. This improves parallelism and reduces the waiting time caused by a shared bus.

4. Single-Bus Organization

In a single-bus system, registers, memory, and I/O devices communicate through one common bus.

Block Diagram

+--------+

|   R1   |

+--------+

|

+--------+

|   R2   |

+--------+

|

v

+---------------------+

|     SINGLE BUS      |

+---------------------+

|       |      |

v       v      v

+-----+  +-----+ +-----+

| ALU |  | MEM | | I/O |

+-----+  +-----+ +-----+

Since all components use the same bus, only one transfer can normally occur on the shared bus during a bus cycle.

5. Multi-Bus Organization

A multi-bus system uses more than one bus to transfer data between different processor components.

Block Diagram

+--------+              +--------+

|   R1   |              |   R2   |

+--------+              +--------+

|                      |

v                      v

+-------------+        +-------------+

|    BUS A    |        |    BUS B    |

+-------------+        +-------------+

|                      |

+----------+-----------+

|

+-----+

| ALU |

+-----+

|

+-----+

| MEM |

+-----+

|

+-----+

| I/O |

+-----+

Multiple buses allow more than one data path to be available. Therefore, data transfers can be performed more efficiently.

6. Components Used

The model consists of:

- Registers R1, R2, R3

- Memory

- I/O device

- ALU

- Single bus

- Multiple buses

- Control unit

- Data transfer operations

7. Data Transfer Operations

The following operations are used to demonstrate data movement.

Register to Register

R2 ← R1

The contents of R1 are transferred to R2.

Memory to Register

R1 ← M[100]

The data stored at memory location 100 is transferred to R1.

Register to Memory

M[200] ← R2

The contents of R2 are transferred to memory location 200.

I/O to Register

R3 ← INPUT

Data received from an input device is transferred to R3.

Register to I/O

OUTPUT ← R3

The contents of R3 are transferred to an output device.

8. Single-Bus Data Transfer Sequence

Consider the following operations:

R1 ← M[100]

R2 ← R1

M[200] ← R2

R3 ← INPUT

OUTPUT ← R3

In a single-bus organization, these operations use the same common bus.

The approximate sequence is:

Step 1: Memory → Bus → R1

Step 2: R1 → Bus → R2

Step 3: R2 → Bus → Memory

Step 4: Input → Bus → R3

Step 5: R3 → Bus → Output

Each transfer must wait for access to the common bus.

9. Multi-Bus Data Transfer Sequence

In a multi-bus system, different buses can be used for different transfers.

For example:

Bus A: Memory → R1

Bus B: R2 → Memory

These transfers can be performed during overlapping time periods if the hardware and control logic allow it.

Another example:

Bus A: INPUT → R3

Bus B: R1 → R2

This demonstrates the advantage of having multiple independent data paths.

10. Algorithm

Single-Bus Simulation

- Start the simulation.

- Initialize registers, memory, and I/O.

- Connect all components to the common bus.

- Perform memory-to-register transfer.

- Perform register-to-register transfer.

- Perform register-to-memory transfer.

- Perform I/O-to-register transfer.

- Perform register-to-I/O transfer.

- Record the number of bus cycles required.

- Calculate the execution time.

- Display the result.

Multi-Bus Simulation

- Start the simulation.

- Initialize registers, memory, and I/O.

- Configure multiple buses.

- Assign data transfers to available buses.

- Perform simultaneous or overlapping transfers where possible.

- Record the number of cycles required.

- Calculate the execution time.

- Compare the result with the single-bus system.

- Display the result.

11. Sample Simulation

Assume the following initial values:

R1 = 10

R2 = 20

R3 = 0

Memory[100] = 50

Memory[200] = 0

Input = 30

Required operations:

1. R1 ← Memory[100]

2. R2 ← R1

3. Memory[200] ← R2

4. R3 ← Input

5. Output ← R3

12. Single-Bus Output

SINGLE BUS SIMULATION

Initial Values:

R1 = 10

R2 = 20

R3 = 0

Memory[100] = 50

Memory[200] = 0

Input = 30

Transfer 1:

Memory[100] → Bus → R1

R1 = 50

Transfer 2:

R1 → Bus → R2

R2 = 50

Transfer 3:

R2 → Bus → Memory[200]

Memory[200] = 50

Transfer 4:

Input → Bus → R3

R3 = 30

Transfer 5:

R3 → Bus → Output

Output = 30

Single-Bus Simulation Completed.

13. Multi-Bus Output

MULTI-BUS SIMULATION

Initial Values:

R1 = 10

R2 = 20

R3 = 0

Memory[100] = 50

Memory[200] = 0

Input = 30

Bus A:

Memory[100] → R1

Bus B:

Input → R3

Bus A:

R1 → R2

Bus B:

R2 → Memory[200]

Bus A:

R3 → Output

Multi-Bus Simulation Completed.

14. Comparison

| Feature |  | Single-Bus |  | Multi-Bus |
| --- | --- | --- | --- | --- |
| Number of buses |  | One |  | Multiple |
| Data transfer |  | Mostly sequential |  | Can be parallel/overlapped |
| Hardware complexity |  | Low |  | Higher |
| Cost |  | Lower |  | Higher |
| Data-transfer speed |  | Lower |  | Higher |
| Bus bottleneck |  | More likely |  | Reduced |
| Parallelism |  | Limited |  | Higher |
| Resource utilization |  | Lower |  | Better |
| Control complexity |  | Simple |  | More complex |

15. Performance Analysis

Suppose the single-bus system requires 5 bus cycles to complete the given operations, while the multi-bus system completes the transfers in 3 effective cycles because some transfers can overlap.

Assume each bus cycle takes 10 ns.

Single-Bus Time

Time = Number of cycles × Cycle time

= 5 × 10 ns

= 50 ns

Multi-Bus Time

Time = 3 × 10 ns

= 30 ns

Speed Improvement

Speedup = Single-Bus Time / Multi-Bus Time

= 50 / 30

= 1.67

Therefore, the multi-bus organization provides approximately 1.67× speedup for this example.

16. Analysis of Results

The simulation shows that the single-bus organization requires all components to share one communication path. As a result, transfers have to wait for bus availability, which can increase execution time and create a bottleneck.

The multi-bus organization provides multiple communication paths. This allows independent transfers to occur in parallel or overlap when the hardware supports it. Therefore, the number of effective cycles can be reduced and the overall data-transfer performance can be improved.

However, the multi-bus system requires additional hardware and more complex control logic. Thus, the improvement in performance comes at the cost of increased system complexity.

17. Result

The single-bus and multi-bus computer organizations were successfully modeled and their data-transfer operations were simulated. The comparison shows that the multi-bus organization can provide better performance and parallel data transfer, while the single-bus organization is simpler and less complex.

18. Conclusion

The experiment demonstrates the difference between single-bus and multi-bus organization. A single bus provides a simple communication structure but may suffer from bottlenecks because multiple components share the same communication path. A multi-bus structure provides additional data paths, allowing greater parallelism and reducing data-transfer delays. Therefore, multi-bus organization is more suitable when higher data-transfer performance is required.

Q3.

1. Aim

To design and simulate a 5-stage instruction pipeline consisting of Fetch (IF), Decode (ID), Execute (EX), Memory (MEM), and Write-back (WB) stages, and compare its performance with a non-pipelined processor using speedup and throughput.

2. Objectives

- To understand the concept of instruction pipelining.

- To simulate a 5-stage instruction pipeline.

- To understand the function of each pipeline stage.

- To execute a sequence of instructions through the pipeline.

- To calculate pipeline speedup.

- To calculate instruction throughput.

- To compare pipelined and non-pipelined execution.

- To analyze the effectiveness of pipelining.

3. Theory

Instruction pipelining is a technique used in processors to improve instruction throughput by dividing instruction execution into multiple stages. Instead of completing one instruction entirely before starting the next instruction, different instructions can occupy different stages at the same time.

The pipeline used in this experiment contains five stages:

- Fetch (IF) – Fetches the instruction from memory.

- Decode (ID) – Decodes the instruction and identifies the required operands.

- Execute (EX) – Performs the arithmetic or logical operation.

- Memory (MEM) – Performs memory access if required.

- Write-back (WB) – Writes the result into the destination register.

The main advantage of pipelining is that after the pipeline is filled, multiple instructions can be processed simultaneously in different stages.

4. Five Pipeline Stages

Stage 1 – Instruction Fetch (IF)

The processor fetches the instruction from instruction memory and places it into the instruction register.

Instruction Memory → Instruction Register

Stage 2 – Instruction Decode (ID)

The fetched instruction is decoded. The processor identifies the operation and the required source and destination registers.

Instruction → Control Unit → Operands

Stage 3 – Execute (EX)

The ALU performs the required operation.

Examples:

ADD

SUB

AND

OR

For an arithmetic instruction:

R3 ← R1 + R2

the addition is performed during the Execute stage.

Stage 4 – Memory Access (MEM)

If the instruction requires memory access, the processor reads from or writes to memory.

For example:

LOAD → Read Memory

STORE → Write Memory

For instructions that do not require memory access, this stage simply passes the result forward.

Stage 5 – Write-back (WB)

The final result is written into the destination register.

Example:

R3 ← ALU Result

5. Pipeline Structure

The basic structure can be represented as:

+-------+    +--------+    +---------+    +--------+    +--------+

Instr → |  IF   | →  |   ID   | →  |   EX    | →  |  MEM   | →  |   WB   |

+-------+    +--------+    +---------+    +--------+    +--------+

Fetch        Decode       Execute       Memory       Write-back

6. Instructions Used for Simulation

Consider the following five instructions:

I1: ADD R1, R2, R3

I2: SUB R4, R5, R6

I3: AND R7, R8, R9

I4: OR  R10, R11, R12

I5: ADD R13, R14, R15

These instructions are selected so that they can pass through the five pipeline stages.

7. Non-Pipelined Execution

In a non-pipelined processor, one instruction must complete all five stages before the next instruction starts.

For five instructions:

I1 → IF → ID → EX → MEM → WB

I2 → IF → ID → EX → MEM → WB

I3 → IF → ID → EX → MEM → WB

I4 → IF → ID → EX → MEM → WB

I5 → IF → ID → EX → MEM → WB

If each stage takes one clock cycle:

Time = Number of instructions × Number of stages

= 5 × 5

= 25 clock cycles

Therefore, the non-pipelined system requires 25 clock cycles.

8. Pipelined Execution

In a pipelined processor, different instructions can occupy different stages during the same clock cycle.

Pipeline Timing Table

| Clock Cycle | IF | ID | EX | MEM | WB |
| --- | --- | --- | --- | --- | --- |
| 1 | I1 | - | - | - | - |
| 2 | I2 | I1 | - | - | - |
| 3 | I3 | I2 | I1 | - | - |
| 4 | I4 | I3 | I2 | I1 | - |
| 5 | I5 | I4 | I3 | I2 | I1 |
| 6 | - | I5 | I4 | I3 | I2 |
| 7 | - | - | I5 | I4 | I3 |
| 8 | - | - | - | I5 | I4 |
| 9 | - | - | - | - | I5 |

Therefore, the five instructions complete in:

9 clock cycles

9. Pipeline Diagram

Cycle →     1    2    3    4    5    6    7    8    9

I1          IF   ID   EX   MEM  WB

I2               IF   ID   EX   MEM  WB

I3                    IF   ID   EX   MEM  WB

I4                         IF   ID   EX   MEM  WB

I5                              IF   ID   EX   MEM  WB

This demonstrates how multiple instructions are processed simultaneously.

10. Algorithm

Non-Pipelined Algorithm

- Start the simulation.

- Load the instruction.

- Perform the Fetch stage.

- Perform the Decode stage.

- Perform the Execute stage.

- Perform the Memory stage.

- Perform the Write-back stage.

- Repeat the process for every instruction.

- Count the total number of clock cycles.

- Display the result.

Pipelined Algorithm

- Start the simulation.

- Load the instructions.

- Divide instruction execution into five stages.

- Fetch the first instruction.

- Decode the first instruction while fetching the second instruction.

- Execute the first instruction while decoding the second and fetching the third.

- Continue overlapping the stages.

- Continue until all instructions complete the Write-back stage.

- Count the total number of clock cycles.

- Calculate speedup and throughput.

- Compare with the non-pipelined system.

- Display the results.

11. Sample Input

Number of Instructions = 5

Instructions:

I1 = ADD R1, R2, R3

I2 = SUB R4, R5, R6

I3 = AND R7, R8, R9

I4 = OR  R10, R11, R12

I5 = ADD R13, R14, R15

Pipeline Stages = 5

Clock Cycle per Stage = 1

12. Sample Pipelined Output

5-STAGE PIPELINE SIMULATION

Stage 1: Fetch

Stage 2: Decode

Stage 3: Execute

Stage 4: Memory

Stage 5: Write-back

Number of Instructions: 5

Pipeline Execution:

Cycle 1: I1-F

Cycle 2: I1-D, I2-F

Cycle 3: I1-E, I2-D, I3-F

Cycle 4: I1-M, I2-E, I3-D, I4-F

Cycle 5: I1-W, I2-M, I3-E, I4-D, I5-F

Cycle 6: I2-W, I3-M, I4-E, I5-D

Cycle 7: I3-W, I4-M, I5-E

Cycle 8: I4-W, I5-M

Cycle 9: I5-W

Total Pipeline Cycles = 9

13. Speedup Calculation

Speedup is calculated as:

Speedup = Non-Pipelined Execution Time

--------------------------------

Pipelined Execution Time

For the given example:

Non-Pipelined Cycles = 25

Pipelined Cycles     = 9

Therefore:

Speedup = 25 / 9

= 2.78

So, the pipelined processor provides approximately 2.78× speedup for this five-instruction example.

14. Throughput Calculation

Throughput represents the number of instructions completed per unit of time.

Assuming one clock cycle is one unit of time:

Throughput = Number of Instructions

-----------------------

Total Execution Cycles

For the pipelined system:

Throughput = 5 / 9

= 0.556 instructions/cycle

Therefore, the pipeline completes approximately 0.556 instructions per clock cycle over the complete five-instruction execution.

Once the pipeline is full, the ideal steady-state throughput approaches one instruction per clock cycle, assuming there are no stalls or hazards.

15. Performance Comparison

| Parameter | Non-Pipelined | Pipelined |
| --- | --- | --- |
| Number of stages | 5 | 5 |
| Number of instructions | 5 | 5 |
| Total cycles | 25 | 9 |
| Speedup | 1× | 2.78× |
| Parallel execution | No | Yes |
| Instruction throughput | Lower | Higher |
| Hardware complexity | Lower | Higher |

16. Analysis of Results

The simulation shows that pipelining significantly reduces the total number of clock cycles required to execute multiple instructions. In the non-pipelined system, each instruction must complete all five stages before the next instruction begins. Therefore, five instructions require 25 clock cycles.

In the pipelined system, different instructions are processed in different stages at the same time. The first instruction takes five cycles to pass through the complete pipeline, but subsequent instructions can enter the pipeline before the previous instruction has finished. As a result, all five instructions complete in only nine cycles.

The calculated speedup is approximately 2.78×, showing that pipelining improves instruction-processing performance. However, the actual performance of a real processor can be lower than the ideal case because of hazards, stalls, branch instructions, and other delays.

17. Result

The 5-stage instruction pipeline was successfully simulated using the stages Fetch, Decode, Execute, Memory, and Write-back. Five instructions were executed using both pipelined and non-pipelined approaches. The pipelined system required 9 cycles, while the non-pipelined system required 25 cycles. The calculated speedup was approximately 2.78×.

18. Conclusion

This experiment demonstrates how instruction pipelining improves processor performance by allowing multiple instructions to be processed simultaneously at different stages. The comparison shows that the pipelined processor provides better throughput and requires fewer clock cycles than the non-pipelined processor for multiple instructions. Thus, pipelining is an important technique for improving instruction execution performance in modern processors.

Q4.

1. Aim

To design and simulate a pipelined processor scenario containing data hazards, control hazards, and structural hazards, and implement techniques such as stalling, data forwarding, and branch prediction to resolve these hazards and improve processor performance.

2. Objectives

- To understand different types of pipeline hazards.

- To identify data hazards in instruction execution.

- To identify control hazards caused by branch instructions.

- To identify structural hazards caused by resource conflicts.

- To understand the effect of hazards on pipeline performance.

- To implement stalling to resolve hazards.

- To implement data forwarding to reduce data-hazard delays.

- To implement branch prediction to reduce control-hazard delays.

- To compare pipeline performance before and after hazard resolution.

3. Theory

A pipeline hazard is a situation that prevents the next instruction from executing in its intended pipeline cycle. Hazards can cause delays, stalls, or incorrect execution if they are not handled properly.

There are three major types of pipeline hazards:

- Data Hazard

- Control Hazard

- Structural Hazard

These hazards reduce the efficiency of a pipeline because instructions may have to wait for required data, decisions, or hardware resources.

4. Types of Pipeline Hazards

4.1 Data Hazard

A data hazard occurs when one instruction depends on the result of a previous instruction that has not yet completed.

Example:

I1: ADD R1, R2, R3

I2: SUB R4, R1, R5

Here, I2 requires the value of R1 produced by I1.

The problem occurs because I2 may try to read R1 before I1 has written the new value.

Data Hazard Diagram

I1: ADD     IF → ID → EX → MEM → WB

I2: SUB          IF → ID → EX → MEM → WB

↑

Needs R1 from I1

5. Data Hazard Resolution Using Stalling

Stalling temporarily stops an instruction from moving forward until the required data becomes available.

Example:

I1: ADD R1, R2, R3

I2: SUB R4, R1, R5

The pipeline can insert a stall between the instructions.

Cycle →  1    2    3    4    5    6    7

I1       IF   ID   EX   MEM  WB

I2            IF   ID   ST   EX   MEM  WB

The ST represents a stall cycle.

The stall ensures that I2 receives the correct value of R1.

6. Data Hazard Resolution Using Data Forwarding

Data forwarding sends the result directly from one pipeline stage to another without waiting for the result to be written back to the register file.

Example:

I1: ADD R1, R2, R3

I2: SUB R4, R1, R5

Instead of waiting for I1 to reach Write-back, the result can be forwarded from the appropriate pipeline stage to I2.

I1: ADD

|

| Result

v

Forwarding Unit

|

v

I2: SUB

This reduces unnecessary pipeline stalls and improves performance.

7. Control Hazard

A control hazard occurs when the processor encounters a branch or jump instruction and does not yet know which instruction should be fetched next.

Example:

I1: ADD R1, R2, R3

I2: BEQ R1, R0, TARGET

I3: SUB R4, R5, R6

I4: AND R7, R8, R9

If the branch is taken, the instructions following the branch may not belong to the correct execution path.

8. Control Hazard Resolution Using Branch Prediction

Branch prediction attempts to predict whether a branch will be taken or not taken.

For example:

Branch instruction:

BEQ R1, R0, TARGET

Prediction:

Branch Taken

If the prediction is correct, the processor continues fetching instructions from the predicted path without waiting.

If the prediction is incorrect, the incorrectly fetched instructions are discarded and execution continues from the correct target.

Simple Branch Prediction

Branch

|

v

Prediction

/       \

Taken    Not Taken

|          |

Fetch      Fetch

Target     Next Instruction

Branch prediction reduces the delay caused by control hazards.

9. Structural Hazard

A structural hazard occurs when two or more instructions require the same hardware resource at the same time.

For example, suppose the processor has only one memory unit that is shared by instruction fetching and data access.

I1: LOAD R1, [100]

I2: ADD R2, R3, R4

If I1 needs memory during the MEM stage while I2 needs the same memory resource for instruction fetching, both operations cannot use the resource simultaneously.

This creates a structural conflict.

10. Structural Hazard Resolution

A structural hazard can be handled by:

- Adding additional hardware resources.

- Separating instruction and data memory.

- Scheduling instructions differently.

- Stalling one instruction when the resource is unavailable.

For example:

Instruction Memory → IF

Data Memory        → MEM

Using separate resources reduces conflicts.

If only one memory is available, a stall may be required:

Cycle →  1    2    3    4    5    6

I1       IF   ID   EX   MEM  WB

I2            IF   ID   ST   EX   MEM  WB

11. Hazard Scenario

The following instruction sequence demonstrates different hazards:

I1: LOAD R1, [100]

I2: ADD R2, R1, R3

I3: BEQ R2, R0, TARGET

I4: SUB R4, R5, R6

I5: AND R7, R8, R9

Hazards Present

Data Hazard:

I1 → I2

I2 requires R1 produced by I1.

Control Hazard:

I3 → Branch TARGET

The processor must determine whether the branch is taken.

Structural Hazard:

A conflict occurs when two pipeline stages require the same hardware resource at the same time.

12. Pipeline Without Hazard Handling

A simplified pipeline execution is:

Cycle →  1    2    3    4    5    6    7    8

I1       IF   ID   EX   MEM  WB

I2            IF   ID   EX   MEM  WB

I3                 IF   ID   EX   MEM  WB

I4                      IF   ID   EX   MEM  WB

I5                           IF   ID   EX   MEM  WB

Without proper hazard detection, dependent instructions may use incorrect or unavailable data, and branch instructions may cause incorrect instruction fetching.

13. Pipeline With Hazard Handling

The processor introduces appropriate mechanisms:

Data Hazard    → Data Forwarding / Stalling

Control Hazard → Branch Prediction

Structural     → Resource Management / Stalling

A simplified execution becomes:

Cycle →  1    2    3    4    5    6    7    8    9

I1       IF   ID   EX   MEM  WB

I2            IF   ID   EX   MEM  WB

I3                 IF   ID   EX   MEM  WB

I4                      IF   ID   EX   MEM  WB

I5                           IF   ID   EX   MEM  WB

Forwarding reduces unnecessary stalls, while branch prediction reduces the waiting time associated with branch decisions.

14. Algorithm

- Start the simulation.

- Initialize the pipeline stages.

- Load the instruction sequence.

- Fetch the first instruction.

- Decode the instruction.

- Check whether a data dependency exists.

- If a data hazard exists, use forwarding or insert a stall.

- Check for branch instructions.

- If a branch is encountered, perform branch prediction.

- Check whether a hardware resource conflict exists.

- If a structural hazard occurs, stall the required instruction or use another available resource.

- Continue instruction execution through EX, MEM, and WB.

- Record the number of stalls.

- Calculate total execution cycles.

- Compare the pipeline with and without hazard handling.

- Display the results.

- Stop the simulation.

15. Sample Input

I1: LOAD R1, [100]

I2: ADD R2, R1, R3

I3: BEQ R2, R0, TARGET

I4: SUB R4, R5, R6

I5: AND R7, R8, R9

Pipeline stages:

IF → ID → EX → MEM → WB

16. Sample Output

PIPELINE HAZARD SIMULATION

Instruction 1: LOAD R1, [100]

Instruction 2: ADD R2, R1, R3

Instruction 3: BEQ R2, R0, TARGET

Instruction 4: SUB R4, R5, R6

Instruction 5: AND R7, R8, R9

Hazard Detection:

Data Hazard:

I1 → I2

Resolution: Data Forwarding

Control Hazard:

I3 → Branch

Resolution: Branch Prediction

Structural Hazard:

Memory Resource Conflict

Resolution: Stall / Resource Separation

Hazards successfully detected and resolved.

Pipeline execution completed.

17. Performance Comparison

Assume the pipeline without hazard handling requires 9 cycles under ideal conditions.

Suppose hazard handling introduces 2 additional stall cycles.

Ideal execution       = 9 cycles

Additional stalls     = 2 cycles

Actual execution      = 11 cycles

Therefore, hazards increase the total execution time.

If forwarding and branch prediction reduce the number of stalls:

Without optimization = 11 cycles

With optimization    = 9 cycles

This demonstrates that hazard-resolution techniques can improve pipeline performance.

18. Comparison of Hazard Resolution Techniques

| Hazard | Problem | Resolution Technique | Benefit |
| --- | --- | --- | --- |
| Data Hazard | Required data is unavailable | Data forwarding / Stalling | Reduces waiting |
| Control Hazard | Branch decision is unknown | Branch prediction | Reduces branch delay |
| Structural Hazard | Hardware resource conflict | Stalling / Additional resources | Avoids resource conflict |

19. Analysis of Results

The simulation demonstrates that pipeline hazards can reduce the performance of a processor. Data hazards occur because instructions may depend on results produced by earlier instructions. Data forwarding can reduce the delay by directly transferring results between pipeline stages, while stalling can be used when forwarding is not sufficient.

Control hazards occur because the processor must determine the correct execution path after a branch instruction. Branch prediction allows the processor to continue execution based on a predicted branch outcome, reducing unnecessary waiting.

Structural hazards occur when multiple instructions require the same hardware resource at the same time. These conflicts can be reduced by providing additional resources or by temporarily stalling an instruction.

Therefore, proper hazard detection and resolution are important for maintaining efficient pipeline execution.

20. Result

The pipeline hazard scenario was successfully designed and analyzed. Data hazards, control hazards, and structural hazards were identified. Data forwarding and stalling were used to handle data dependencies, branch prediction was used to reduce control-hazard delays, and resource management/stalling was used to handle structural hazards.

21. Conclusion

This experiment demonstrates the major hazards that can occur in a pipelined processor and their effects on instruction execution. Data hazards can be reduced using forwarding and stalling, control hazards can be reduced using branch prediction, and structural hazards can be handled through resource management or additional hardware. These techniques help reduce pipeline delays and improve overall processor performance.

Q5.

1. Aim

To analyze a processor supporting superscalar execution, out-of-order execution, speculative execution, Hyper-Threading, and Simultaneous Multithreading (SMT), and evaluate how these techniques improve instruction throughput and processor resource utilization.

2. Objectives

- To understand superscalar processor architecture.

- To understand how multiple instructions can execute in parallel.

- To study out-of-order execution.

- To study speculative execution.

- To understand Hyper-Threading.

- To understand Simultaneous Multithreading (SMT).

- To compare instruction throughput using different execution techniques.

- To analyze processor resource utilization.

- To understand the advantages and limitations of parallel instruction execution.

3. Theory

A superscalar processor is a processor that can issue and execute more than one instruction during a single clock cycle. Instead of processing only one instruction at a time, the processor uses multiple execution units to execute independent instructions in parallel.

For example, if two instructions are independent:

I1: ADD R1, R2, R3

I2: SUB R4, R5, R6

both instructions may be executed at the same time if the processor has sufficient execution resources.

Superscalar execution improves instruction throughput because more instructions can be processed in a given amount of time.

4. Superscalar Architecture

A simplified superscalar processor can be represented as:

+----------------+

| Instruction    |

|     Fetch      |

+-------+--------+

|

v

+----------------+

| Instruction    |

|     Decode     |

+-------+--------+

|

v

+----------------+

| Instruction    |

|     Issue      |

+-------+--------+

|

+----------+----------+

|          |          |

v          v          v

+-------+  +-------+  +-------+

| ALU 1 |  | ALU 2 |  | Load/ |

|       |  |       |  | Store |

+-------+  +-------+  +-------+

|          |          |

+----------+----------+

|

v

+----------------+

| Write Back     |

+----------------+

Multiple execution units allow independent instructions to execute simultaneously.

5. Superscalar Execution

Consider four independent instructions:

I1: ADD R1, R2, R3

I2: SUB R4, R5, R6

I3: AND R7, R8, R9

I4: OR  R10, R11, R12

In a single-issue processor, the instructions may be issued one at a time.

In a 2-way superscalar processor, two instructions can potentially be issued per cycle:

Cycle 1 → I1 + I2

Cycle 2 → I3 + I4

Thus, four independent instructions can be issued in approximately two issue cycles, subject to the processor's resources and dependencies.

6. Out-of-Order Execution

Out-of-order execution allows instructions to execute when their required operands and execution resources are available rather than strictly following their original order.

Consider:

I1: LOAD R1, [100]

I2: ADD R2, R1, R3

I3: SUB R4, R5, R6

I2 depends on I1, so it must wait for the required value of R1.

However, I3 is independent of I1 and I2. Therefore, the processor can execute I3 while I1 is waiting for the memory operation to complete.

Original Order:

I1 → I2 → I3

Possible Execution:

I1 → waiting

I3 → Execute

I2 → Execute after R1 becomes available

This improves utilization of execution units because independent instructions do not have to wait unnecessarily.

7. Speculative Execution

Speculative execution allows a processor to execute instructions before it is certain that those instructions are on the correct execution path.

This is particularly useful for branch instructions.

Example:

I1: ADD R1, R2, R3

I2: BEQ R1, R0, TARGET

I3: SUB R4, R5, R6

I4: AND R7, R8, R9

When the processor encounters the branch, it may predict the branch direction and begin executing instructions from the predicted path.

If the prediction is correct:

Prediction Correct

↓

Continue Execution

↓

No major pipeline interruption

If the prediction is incorrect, the speculative instructions are discarded and execution continues from the correct path.

8. Hyper-Threading

Hyper-Threading is a processor technology that allows one physical processor core to present multiple logical processors to the operating system.

The logical processors share many physical resources of the core but can maintain separate architectural states.

A simplified representation is:

Physical Core

|

+-------+-------+

|               |

Thread 1         Thread 2

|               |

+-------+-------+

|

Shared Execution

Resources

When one thread cannot fully use the processor resources, another thread may use available resources.

This can improve overall processor utilization and throughput for suitable workloads.

9. Simultaneous Multithreading (SMT)

Simultaneous Multithreading (SMT) allows instructions from multiple threads to be issued and executed during the same processor cycle when sufficient resources are available.

For example:

Thread 1:

I1: ADD

I2: SUB

Thread 2:

I3: AND

I4: OR

The processor may issue instructions from both threads during the same cycle:

Cycle 1 → Thread 1: ADD + Thread 2: AND

Cycle 2 → Thread 1: SUB + Thread 2: OR

This improves utilization of execution units when one thread alone cannot keep all resources busy.

10. Difference Between Superscalar and SMT

| Feature | Superscalar | SMT |
| --- | --- | --- |
| Main idea | Multiple instructions per cycle | Instructions from multiple threads per cycle |
| Parallelism | Instruction-level | Thread-level + instruction-level |
| Threads | Can operate with one thread | Multiple threads |
| Main benefit | Higher instruction throughput | Better resource utilization |
| Resource sharing | Execution units used by instructions | Execution resources shared between threads |

11. Combined Processor Model

The concepts can be combined into a simplified processor model:

+------------------+

| Instruction      |

| Fetch / Branch   |

| Prediction       |

+--------+---------+

|

v

+------------------+

| Decode / Rename  |

+--------+---------+

|

v

+------------------+

| Instruction      |

| Scheduling       |

+--------+---------+

|

+-----------+-----------+

|                       |

v                       v

+-------------+         +-------------+

| Thread 1    |         | Thread 2    |

+------+------+         +------+------+

|                       |

+-----------+-----------+

|

v

+------------------+

| Execution Units  |

| ALU / Load/Store |

+--------+---------+

|

v

+------------------+

| Write Back       |

+------------------+

This model demonstrates how superscalar execution, out-of-order execution, speculative execution, and SMT can work together.

12. Algorithm

- Start the simulation.

- Initialize the processor resources.

- Create a set of independent and dependent instructions.

- Fetch the instructions.

- Decode the instructions.

- Identify instruction dependencies.

- Issue independent instructions to available execution units.

- Allow multiple instructions to execute when resources are available.

- Use out-of-order execution for independent instructions.

- Detect branch instructions.

- Apply speculative execution using a predicted branch direction.

- Create multiple threads for SMT simulation.

- Issue instructions from different threads when resources are available.

- Record the number of completed instructions.

- Calculate instruction throughput.

- Measure processor resource utilization.

- Compare normal and advanced execution.

- Display the results.

- Stop the simulation.

13. Sample Input

Consider the following instructions:

I1: ADD R1, R2, R3

I2: SUB R4, R5, R6

I3: LOAD R7, [100]

I4: AND R8, R9, R10

I5: BEQ R1, R0, TARGET

I6: OR R11, R12, R13

For SMT, assume two threads:

Thread 1:

I1, I2, I3

Thread 2:

I4, I5, I6

14. Sample Execution

Superscalar Execution

Assume a 2-way issue processor:

Cycle 1 → I1 + I2

Cycle 2 → I3 + I4

Cycle 3 → I5 + I6

Multiple instructions are issued during each cycle when resources and dependencies permit.

Out-of-Order Execution

Suppose I3 is waiting for memory:

I1 → Execute

I2 → Execute

I3 → Waiting for Memory

I4 → Execute

The independent I4 instruction does not have to wait for I3.

Speculative Execution

For I5:

I5: BEQ R1, R0, TARGET

Prediction: NOT TAKEN

The processor begins fetching the next sequential instruction.

If the prediction is correct, execution continues normally.

15. Sample Output

SUPERSCALAR PROCESSOR SIMULATION

Number of Threads: 2

Issue Width: 2

Superscalar Execution:

Cycle 1: I1, I2

Cycle 2: I3, I4

Cycle 3: I5, I6

Out-of-Order Execution:

Independent instructions executed without waiting.

Speculative Execution:

Branch prediction applied to I5.

Prediction: NOT TAKEN

SMT:

Thread 1 and Thread 2 executed using shared processor resources.

Simulation Completed Successfully.

16. Performance Measurement

Assume a simple processor executes 6 instructions.

Single-Issue Execution

If one instruction is issued per cycle:

Instructions = 6

Cycles = 6

Throughput = 6 / 6

= 1 instruction/cycle

2-Way Superscalar Execution

If two instructions can be issued per cycle:

Instructions = 6

Ideal issue cycles = 6 / 2

= 3 cycles

Throughput = 6 / 3

= 2 instructions/cycle

Therefore, under ideal conditions, the 2-way superscalar processor provides approximately 2 instructions per cycle, compared with 1 instruction per cycle for the single-issue processor.

17. Resource Utilization

Consider four execution resources:

ALU 1

ALU 2

Load/Store Unit

Branch Unit

Without advanced techniques, some resources may remain unused while an instruction is waiting.

With out-of-order execution and SMT, instructions from other independent operations or threads can use available resources.

Example:

ALU 1             → 80% utilized

ALU 2             → 75% utilized

Load/Store Unit   → 65% utilized

Branch Unit       → 50% utilized

Average utilization:

Average Utilization

= (80 + 75 + 65 + 50) / 4

= 67.5%

This is an example measurement for the simulation; actual values depend on the instruction workload and processor model.

18. Performance Comparison

| Technique | Main Advantage | Effect on Throughput | Resource Utilization |
| --- | --- | --- | --- |
| Basic execution | Simple execution | Low | Lower |
| Superscalar | Multiple instructions per cycle | Higher | Higher |
| Out-of-order | Executes ready instructions early | Higher | Better |
| Speculative execution | Reduces branch waiting | Higher | Better |
| Hyper-Threading | Uses multiple logical threads | Higher | Better |
| SMT | Issues instructions from multiple threads | Higher | Higher |

19. Analysis of Results

The simulation demonstrates that superscalar execution increases instruction throughput by allowing multiple instructions to be issued and executed in parallel. Out-of-order execution further improves performance by allowing independent instructions to execute when their required operands and resources are available instead of forcing them to wait for earlier instructions.

Speculative execution helps reduce the performance impact of branch instructions by allowing the processor to continue executing predicted instructions before the branch outcome is confirmed. Correct predictions can reduce delays, while incorrect predictions require the speculative work to be discarded.

Hyper-Threading and SMT improve processor resource utilization by allowing multiple logical threads to share the resources of a physical processor core. When one thread cannot fully utilize the available resources, instructions from another thread can use those resources.

20. Result

The superscalar processor model was successfully analyzed with out-of-order execution, speculative execution, Hyper-Threading, and SMT. The analysis shows that executing multiple independent instructions and using multiple threads can increase instruction throughput and improve utilization of processor resources.

21. Conclusion

This experiment demonstrates how modern processors improve performance through different forms of parallel execution. Superscalar execution allows multiple instructions to execute in parallel, while out-of-order execution ensures that ready instructions can use available resources efficiently. Speculative execution reduces the impact of branch decisions, and Hyper-Threading/SMT allows multiple threads to share processor resources. Together, these techniques can significantly improve instruction throughput and resource utilization compared with simple sequential execution.