SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | N.Nesarajan | Reg.No.: | 192511386 |
| Department: | CSE | Date: | 24-08-2026 |

ANNEXURE A

COMPARATIVE ANALYSIS

- Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

- Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

- Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

- Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

- Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

Code of Conduct:

I N.Nesarajan/192511386 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

ANSWERS

Q1. Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

Answer

Cache memory is a high-speed memory used by the processor to store frequently accessed data and instructions. Modern processors generally use three levels of cache: L1, L2, and L3. They differ mainly in their size, latency, bandwidth, and throughput.

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Size | Smallest | Larger than L1 | Largest |
| Latency | Very low | Low | Higher than L1 and L2 |
| Bandwidth | Very high | High | Moderate |
| Throughput | Very high | High | Moderate |
| Location | Closest to CPU core | Near CPU core | Usually shared among cores |
| Speed | Fastest | Fast | Slower than L1 and L2 |
| Main purpose | Frequently used data/instructions | Backup for L1 | Backup for L2 and shared data |

L1 Cache

L1 is the smallest and fastest cache. It is located very close to the processor core and normally contains separate instruction and data caches. Due to its very low latency and high bandwidth, the CPU can quickly access frequently required information. A high L1 cache hit rate improves processor performance by reducing waiting time.

L2 Cache

L2 is larger than L1 but has slightly higher latency. It stores data that is not available in L1 and therefore acts as a second-level cache. Although it is slower than L1, it is still much faster than accessing main memory. L2 reduces the number of RAM accesses and helps maintain good processor throughput.

L3 Cache

L3 is larger than L1 and L2 and is commonly shared by multiple processor cores. It has higher latency and lower bandwidth compared with L1 and L2. However, its larger capacity allows it to store more frequently used data. It is especially useful in multicore processors because multiple cores can access shared data.

Impact on Processor Performance

The cache levels work together as a memory hierarchy. When the processor needs data, it first checks L1. If the data is not found, it checks L2 and then L3 before accessing main memory. Therefore, finding data in a higher-level cache reduces memory access latency.

A high cache hit rate improves CPU throughput because the processor spends less time waiting for data. A low cache hit rate causes more accesses to slower main memory, increasing latency and reducing performance. Thus, L1 provides the fastest access, L2 provides additional capacity with slightly higher latency, and L3 provides large shared storage to reduce expensive main-memory accesses.

Conclusion

L1, L2, and L3 caches provide different balances between latency, bandwidth, capacity, and throughput. L1 has the lowest latency and highest bandwidth, L2 provides greater capacity with slightly higher latency, and L3 provides the largest cache capacity but with higher latency. Together, they significantly improve processor performance by reducing memory access time and increasing overall system throughput.

Q2. Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

Answer

SRAM (Static Random Access Memory) and DRAM (Dynamic Random Access Memory) are two important types of semiconductor memory used in computer systems. SRAM is mainly used for cache memory, while DRAM is mainly used for main memory (RAM). They differ in their speed, storage capacity, cost, latency, bandwidth, and throughput.

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Full Form | Static Random Access Memory | Dynamic Random Access Memory |
| Latency | Very low | Higher than SRAM |
| Bandwidth | High | High, but generally higher access latency |
| Throughput | Very high | High |
| Speed | Faster | Slower than SRAM |
| Storage Capacity | Smaller | Larger |
| Cost | Expensive | Less expensive |
| Refresh | Not required | Required periodically |
| Main Application | Cache memory | Main memory |

SRAM

SRAM stores each bit using a circuit of transistors and does not require periodic refreshing while power is supplied. This allows SRAM to provide very low latency and fast access. Its high speed and throughput make it suitable for processor cache levels such as L1, L2, and L3. However, SRAM requires more physical space per bit and is therefore expensive and limited in capacity.

DRAM

DRAM stores data using a capacitor and transistor. Since the stored charge gradually disappears, DRAM requires periodic refresh operations to maintain the stored information. This results in higher access latency compared with SRAM. However, DRAM has a much higher storage density and lower cost per bit, making it suitable for large main-memory systems.

Latency Comparison

SRAM has lower latency because its storage cells can be accessed quickly without refresh operations during normal access. DRAM has higher latency because accessing data involves additional operations and periodic refresh requirements. Therefore, SRAM is preferred when the processor needs extremely fast data access.

Bandwidth and Throughput

SRAM provides very high bandwidth and throughput for small amounts of frequently accessed data. This makes it ideal for cache memory, where the processor needs rapid access to instructions and data. DRAM can provide substantial bandwidth for transferring larger amounts of data, but its higher access latency makes it less suitable for processor cache. Modern DRAM systems use techniques such as burst transfers and multiple memory channels to improve effective bandwidth and throughput.

Use in Cache and Main Memory Design

The main difference in their use is based on the trade-off between speed and capacity. SRAM is used in cache because processors require very low latency and high throughput. DRAM is used as main memory because computer systems need a much larger memory capacity at a reasonable cost.

Conclusion

SRAM and DRAM serve different purposes in the memory hierarchy. SRAM provides lower latency and very high throughput, making it suitable for cache memory. DRAM provides higher capacity at lower cost, making it suitable for main memory despite its comparatively higher latency. Therefore, using SRAM for cache and DRAM for main memory provides a practical balance between processor performance, memory capacity, and cost.

Q3. Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

Answer

Virtual memory, paging, and cache memory are important mechanisms used in computer architecture to improve memory management and system performance. Although they all deal with memory access, their purposes are different. Virtual memory provides a larger logical address space, paging manages the movement of memory pages, and cache memory provides very fast access to frequently used data.

| Feature | Virtual Memory | Paging | Cache Memory |
| --- | --- | --- | --- |
| Main Purpose | Provides a larger logical memory space | Manages memory in fixed-size pages | Speeds up frequently accessed data |
| Storage Used | RAM + secondary storage | RAM + secondary storage | SRAM/cache storage |
| Access Latency | High when secondary storage is accessed | High during page faults | Very low |
| Normal Access Speed | Slower than RAM | Depends on page availability | Fastest |
| Throughput | Can decrease during frequent swapping | Decreases when page faults occur | Improves CPU throughput |
| Main Benefit | Allows programs larger than physical RAM | Efficient memory allocation | Reduces CPU memory access time |

Virtual Memory

Virtual memory allows the operating system to use a portion of secondary storage as an extension of physical RAM. It provides each program with a larger logical address space than the available physical memory. When required data is already in RAM, access is relatively fast. However, if the required data must be retrieved from secondary storage, the access latency becomes very high.

Paging

Paging divides virtual memory and physical memory into fixed-size blocks called pages and frames. It allows the operating system to move required pages between secondary storage and RAM. When a required page is not present in RAM, a page fault occurs. The operating system must retrieve the page from secondary storage, which significantly increases latency and can reduce system throughput.

Cache Memory

Cache memory is a small, high-speed memory located close to the processor. It stores frequently or recently accessed instructions and data. When the required information is found in the cache, the CPU can access it much faster than main memory. Therefore, cache memory has very low access latency and helps increase processor throughput.

Access Latency Comparison

Cache memory has the lowest access latency because it is designed for very fast processor access. Virtual memory and paging normally have higher latency because they involve memory management and may require secondary-storage access. A cache hit can be completed quickly, whereas a page fault can require a much slower transfer from secondary storage.

Effect on System Throughput

Cache memory generally improves throughput because the CPU spends less time waiting for data. Virtual memory and paging allow efficient use of memory and support larger programs, but frequent page faults can reduce throughput because the processor may have to wait for data to be transferred from secondary storage.

Example

Consider a program that frequently accesses the same data. If the data is stored in cache, the processor can retrieve it quickly, resulting in high throughput. If the data is in RAM but not in cache, the processor experiences higher latency. If the required page is not in RAM, a page fault occurs and the operating system must load it from secondary storage, causing significantly higher latency.

Conclusion

Cache memory primarily focuses on reducing access latency and increasing processor throughput, while virtual memory and paging focus on efficient memory management and providing a larger logical memory space. Cache provides the fastest access, whereas paging and virtual-memory operations can introduce significant delays when page faults occur. Therefore, these mechanisms work together as different levels of the memory-management hierarchy to balance performance, capacity, and efficient memory utilization.

Q4. Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

Answer

NAND Flash and DRAM are both important memory technologies, but they have different characteristics and are used at different levels of a computer system. DRAM is mainly used as main memory, while NAND Flash is commonly used for secondary storage such as SSDs and flash-based storage devices.

| Feature | NAND Flash | DRAM |
| --- | --- | --- |
| Data Storage | Non-volatile | Volatile |
| Latency | Higher | Much lower |
| Bandwidth | High for storage devices, but access is slower | High |
| Throughput | Good for large data transfers | Very high for active program data |
| Refresh | Not required | Required |
| Power Loss | Retains data | Loses stored data |
| Main Use | SSDs, memory cards, USB storage | Main memory |
| Hierarchical Position | Secondary storage | Main memory |

NAND Flash

NAND Flash is a non-volatile memory technology, meaning that it can retain data even when power is removed. It is widely used in SSDs, memory cards, and other storage devices. NAND Flash provides good sequential data-transfer performance, but its access latency is much higher than DRAM. It is therefore not normally used as the processor's main working memory.

DRAM

DRAM is a volatile memory technology that is primarily used as main memory. It provides much lower access latency than NAND Flash and supports high-speed access to the data and instructions currently being used by the processor. However, DRAM requires periodic refreshing because the charge stored in its memory cells gradually leaks away.

Latency Comparison

The major difference between NAND Flash and DRAM is their access latency. DRAM has significantly lower latency, allowing the processor to access active data quickly. NAND Flash has higher latency because it is designed primarily for persistent storage rather than extremely fast CPU memory access.

When the processor needs data that is already available in DRAM, access can occur relatively quickly. If data must be retrieved from NAND Flash storage, the additional storage access delay can significantly increase overall memory-access time.

Bandwidth and Throughput

DRAM provides high bandwidth for continuously transferring data between the processor and main memory. This helps maintain high system throughput during program execution. NAND Flash can also provide high sequential bandwidth, especially in modern SSDs, but its access characteristics are less suitable for the small, frequent accesses required by the CPU.

Therefore, DRAM is better suited for high-frequency active data access, while NAND Flash is more suitable for storing large amounts of persistent data.

Suitability for Hierarchical Memory Systems

Both technologies are useful in a hierarchical memory system because they provide different balances between speed, capacity, cost, and persistence.

A typical hierarchy can be represented as:

CPU Registers → Cache → DRAM → NAND Flash Storage

DRAM is placed closer to the processor because of its lower latency and high bandwidth. NAND Flash is placed further away because it provides much larger non-volatile storage capacity but with higher access latency.

Critical Analysis

Using only DRAM would provide fast access but would be expensive for storing very large amounts of data and would lose data when power is removed. Using only NAND Flash would provide large persistent storage, but its higher latency would negatively affect active program execution. Combining both technologies provides a practical balance: DRAM handles frequently accessed working data, while NAND Flash provides large and persistent storage.

Conclusion

NAND Flash and DRAM have different roles in hierarchical memory systems. DRAM provides lower latency and high bandwidth, making it suitable for main memory and active program data. NAND Flash provides persistent storage with larger capacity but higher access latency, making it suitable for secondary storage. Their combination creates an efficient memory hierarchy that balances performance, capacity, cost, and data persistence.

Q5. Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

Answer

MRAM (Magnetoresistive Random Access Memory) and RRAM (Resistive Random Access Memory) are advanced non-volatile memory technologies. Both can retain data without continuous power and are being considered for applications where conventional memory technologies may have limitations. They differ in their access time, throughput, energy consumption, endurance, and implementation characteristics.

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Full Form | Magnetoresistive Random Access Memory | Resistive Random Access Memory |
| Data Storage | Magnetic state | Resistance state |
| Access Time | Very fast | Fast |
| Throughput | High | High |
| Energy Efficiency | High | Very high potential |
| Volatility | Non-volatile | Non-volatile |
| Data Retention | Good | Good |
| Main Advantage | Fast access and high endurance | High density and low-power operation |

MRAM

MRAM stores information using magnetic states rather than electrical charge. It is non-volatile, so stored information remains available even when power is removed. MRAM provides fast access and can achieve high throughput, making it suitable for applications that require quick read and write operations.

One important advantage of MRAM is its combination of speed, non-volatility, and endurance. It can be used in embedded systems and other applications where frequent memory operations are required.

RRAM

RRAM stores information by changing the resistance of a memory cell between different states. It is also non-volatile and can potentially provide high memory density. RRAM is attractive for low-power applications because memory operations can be performed with relatively low energy consumption.

RRAM can provide fast access and good throughput, while its simple cell structure has the potential to support high-density memory systems. However, practical RRAM implementations can have challenges related to device variability and endurance.

Access Time Comparison

Both MRAM and RRAM are designed to provide relatively fast access compared with many traditional non-volatile storage technologies. MRAM is particularly attractive when fast and predictable access is required, while RRAM also offers fast access and has potential for further improvements as the technology develops.

Lower access time means that the processor spends less time waiting for memory operations. Therefore, both technologies can improve system responsiveness when appropriately integrated into the memory hierarchy.

Throughput Comparison

Throughput represents the amount of data that can be processed or transferred during a given period. MRAM can provide high throughput because of its fast read and write operations. RRAM can also provide high throughput and is attractive for dense memory architectures.

The actual throughput of either technology depends on factors such as memory architecture, interface design, operating conditions, and read/write characteristics.

Energy Efficiency

Energy efficiency is an important advantage of emerging non-volatile memory technologies. MRAM does not require continuous power to retain stored information, which can reduce standby power consumption. However, the energy required for writing can depend on the particular MRAM technology.

RRAM also has strong potential for low-energy operation, particularly because changing the resistance state can be performed with relatively small energy requirements. This makes RRAM attractive for energy-sensitive systems and future high-density memory applications.

Critical Analysis

MRAM provides a strong combination of fast access, high throughput, non-volatility, and endurance, making it suitable for applications requiring frequent memory operations. RRAM offers high density and strong energy-efficiency potential, making it attractive for low-power and large-capacity memory systems.

Therefore, the choice between MRAM and RRAM depends on the application's requirements. Systems prioritizing speed and endurance may benefit from MRAM, while systems prioritizing density and low energy consumption may find RRAM more suitable.

Conclusion

MRAM and RRAM are promising advanced non-volatile memory technologies. MRAM offers fast access, high throughput, and good endurance, while RRAM provides fast operation with strong potential for high density and energy efficiency. Both technologies can play an important role in future hierarchical memory systems by providing a balance between performance, persistence, and power consumption.