SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | N.Nesarajan | Reg.No.: | 192511386 |
| Department: | CSE | Date: | 26-08-2026 |

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

Code of Conduct:

I ____________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (25%) | Constraint Analysis (25%) | Solution Development (25%) | Application of Concepts (15%) | Justification (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |

Answers

Q1. Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

• CPU utilization must be below 15%
• Multiple sensors generate interrupts every 1 ms
• Use vectored interrupt handling
• Select between memory-mapped and I/O-mapped I/O for optimal latency

ANSWER:

- Problem Understanding

A real-time embedded system receives data from multiple sensors at regular intervals. In this system, multiple sensors generate interrupts every 1 ms. Therefore, the system must respond to sensor interrupts quickly while keeping CPU utilization below 15%.

To achieve fast interrupt response, vectored interrupt handling is used. Each sensor is assigned a separate interrupt vector so that the CPU can directly identify the source of the interrupt. The I/O technique must also be selected carefully to reduce communication latency. Memory-mapped I/O is selected because it provides simple and efficient access to sensor registers.

- Constraint Analysis

The major constraints of the system are:

- CPU utilization must remain below 15%.

- Multiple sensors generate interrupts every 1 ms.

- Vectored interrupt handling must be used.

- The I/O technique must provide low latency.

- The interrupt service routine should be short to reduce CPU overhead.

Since interrupts are generated every 1 ms, the CPU may become overloaded if each interrupt requires lengthy processing. Therefore, only essential operations should be performed inside the Interrupt Service Routine (ISR). The remaining processing can be performed as a background task.

- Proposed Architecture

The proposed architecture is:

Sensors
↓
Vectored Interrupt Controller
↓
CPU
↓
Memory-Mapped I/O
↓
Sensor Registers
↓
Main Memory Buffer
↓
Background Processing

Detailed Architecture:

+----------------+

|    Sensor 1    |

+-------+--------+

|

| Interrupt

|

+-------v--------+

|    Sensor 2    |

+-------+--------+

|

| Interrupt

|

+-------v--------+

|    Sensor N    |

+-------+--------+

|

v

+------------------------+

| Vectored Interrupt     |

| Controller             |

+-----------+------------+

|

| Interrupt Vector

v

+-----------------+

|       CPU       |

|    Short ISR    |

+--------+--------+

|

| Memory-Mapped I/O

v

+-----------------+

| Sensor Registers|

+--------+--------+

|

v

+-----------------+

|   Main Memory   |

|   Data Buffer   |

+--------+--------+

|

v

+-----------------+

| Background Data |

|   Processing    |

+-----------------+

4. Working of the Proposed System

First, the sensors continuously monitor the required parameters. Whenever a sensor has new data, it generates an interrupt.

The interrupt controller receives the interrupt request and identifies the source of the interrupt. Since vectored interrupt handling is used, each sensor has a unique interrupt vector.

The CPU receives the interrupt vector and directly jumps to the corresponding Interrupt Service Routine. The ISR reads the sensor data, stores it in a memory buffer, acknowledges the interrupt, and then returns to the previous task.

The actual data processing is performed as a background task. This prevents the CPU from spending a long time inside the ISR and helps to keep CPU utilization below 15%.

5.Selection of I/O Technique

Two I/O techniques can be considered:

Memory-Mapped I/O:

In Memory-Mapped I/O, device registers are assigned addresses within the processor's memory address space. The CPU can access these registers using normal memory read and write instructions.

Advantages:

• Simple and efficient register access.
• Uses normal memory instructions.
• Easy integration with the processor.
• Suitable for fast sensor communication.
• Provides low communication latency.

I/O-Mapped I/O:

In I/O-Mapped I/O, the I/O devices use a separate I/O address space. Special I/O instructions are generally required to communicate with the devices.

Selection:

Memory-Mapped I/O is selected for this system because it provides simple and efficient access to sensor registers and is suitable for achieving low communication latency.

6.Interrupt Handling Mechanism

The system uses vectored interrupts for handling sensor interrupts.

When an interrupt is generated:

- The sensor generates an interrupt request.

- The interrupt controller receives the request.

- The interrupt controller identifies the sensor.

- The corresponding interrupt vector is supplied to the CPU.

- The CPU directly executes the appropriate ISR.

- The ISR reads and stores the sensor data.

- The interrupt is acknowledged.

- The CPU returns to the previous task.

- Further data processing is performed in the background.

- CPU Utilization Optimization

To maintain CPU utilization below 15%, the following techniques are used:

• Keep the Interrupt Service Routine short.
• Perform only essential operations inside the ISR.
• Store sensor data in a memory buffer.
• Perform complex processing outside the ISR.
• Use vectored interrupts to reduce interrupt identification time.
• Use efficient Memory-Mapped I/O for sensor register access.

These techniques reduce the amount of CPU time spent handling frequent sensor interrupts.

8.Application of Computer Architecture Concepts

The proposed system applies the following computer architecture concepts:

• Vectored Interrupts – Used to quickly identify the source of an interrupt.
• Memory-Mapped I/O – Used to access sensor registers efficiently.
• Interrupt Controller – Used to manage interrupts from multiple sensors.
• Interrupt Service Routine – Used to handle sensor interrupts.
• Memory Buffer – Used to temporarily store sensor data.
• Background Processing – Used to perform non-critical processing without increasing ISR execution time.

9.Justification

Memory-Mapped I/O is selected because the system requires efficient access to sensor registers and low communication latency. Vectored interrupts are used because multiple sensors generate interrupts every 1 ms, making fast interrupt identification necessary.

The Interrupt Service Routine is kept short so that the CPU does not spend excessive time handling interrupts. Sensor data is stored in memory and processed later as a background task.

Therefore, the proposed mechanism satisfies the requirements of low latency, vectored interrupt handling, frequent sensor interrupts, and CPU utilization below 15%.

10.Conclusion

The proposed real-time I/O communication mechanism uses Memory-Mapped I/O, vectored interrupt handling, short Interrupt Service Routines, memory buffering, and background processing.

This architecture provides fast communication between the sensors and CPU while reducing CPU overhead. The design is suitable for a real-time embedded system because it provides low interrupt latency and is designed to maintain CPU utilization below 15%.

Q2. Design an I/O subsystem under the following constraints:

• High-speed storage requires throughput greater than 3 GB/s.
• Must use NVMe over PCIe.
• CPU should not handle bulk data transfer.
• Integrate DMA controller with interrupt notification.

ANSWER:

- Problem Understanding

The problem is to design a high-speed I/O subsystem for storage devices that requires a throughput greater than 3 GB/s. The system must use NVMe technology over the PCIe interface.

Since the storage device transfers a large amount of data, the CPU should not be responsible for handling the bulk data transfer. Therefore, a DMA controller is integrated to transfer data directly between the NVMe storage device and main memory.

An interrupt notification mechanism is also required so that the CPU is informed when the DMA transfer or storage operation is completed.

- Constraint Analysis

The major constraints of the system are:

- Storage throughput must be greater than 3 GB/s.

- NVMe must be used as the storage communication protocol.

- PCIe must be used as the communication interface.

- The CPU should not handle bulk data transfers.

- DMA must be used for direct data transfer.

- An interrupt notification mechanism must inform the CPU after the transfer is completed.

The main priority is high-speed data transfer with minimum CPU involvement. Therefore, NVMe over PCIe combined with DMA is selected.

- Proposed Architecture

The proposed I/O subsystem architecture is:

+----------------+

|      CPU       |

+-------+--------+

|

Commands / Setup

|

v

+----------------+

| PCIe Root      |

| Complex        |

+-------+--------+

|

| PCIe

|

v

+----------------+

| NVMe Controller|

+-------+--------+

|

| NVMe

|

v

+----------------+

| NVMe SSD       |

| High-Speed     |

| Storage        |

+-------+--------+

|

| Data Transfer

v

+----------------+

| DMA Controller |

+-------+--------+

|

| Direct Data Transfer

v

+----------------+

|  Main Memory   |

+----------------+

DMA / NVMe

|

| Interrupt

v

+----------------+

| Interrupt      |

| Controller     |

+-------+--------+

|

v

CPU

4. Working of the Proposed System

First, the CPU initializes the NVMe controller and configures the required storage operation.

The CPU communicates with the NVMe controller through the PCIe interface. The NVMe controller provides high-speed communication with the NVMe SSD.

When a large amount of data needs to be transferred, the DMA controller is configured to perform the transfer.

The DMA controller transfers data directly between the NVMe storage device and main memory without requiring the CPU to copy each block of data.

During the bulk transfer, the CPU is free to perform other processing tasks.

After the DMA transfer is completed, the DMA controller or NVMe controller generates an interrupt notification.

The CPU receives the interrupt and executes the appropriate interrupt service routine to determine that the data transfer has been completed.

- Role of PCIe

PCIe is used as the high-speed communication interface between the system and the NVMe storage controller.

PCIe provides high-speed and efficient communication and is suitable for storage systems requiring throughput greater than 3 GB/s.

The PCIe Root Complex connects the processor and memory system to the NVMe controller.

- Role of NVMe

NVMe stands for Non-Volatile Memory Express.

NVMe is designed for high-speed storage devices such as SSDs. It provides an efficient command and communication mechanism for accessing non-volatile storage.

In this system, NVMe is used to communicate with the high-speed SSD through the PCIe interface.

- Role of DMA Controller

DMA stands for Direct Memory Access.

The DMA controller allows data to be transferred directly between the storage device and main memory without requiring the CPU to handle every individual data transfer.

The basic DMA operation is:

- CPU configures the DMA controller.

- CPU specifies the source, destination, and transfer size.

- DMA starts the data transfer.

- Data is transferred directly between the device and memory.

- CPU continues performing other tasks.

- DMA generates an interrupt after completion.

- CPU handles the completion interrupt.

- Interrupt Notification

The interrupt mechanism is used to notify the CPU when the data transfer is completed.

After completing the DMA operation, the DMA controller generates an interrupt request.

The CPU receives the interrupt and executes the corresponding Interrupt Service Routine. The ISR checks the transfer status and allows the system to continue with the next operation.

This approach prevents the CPU from continuously checking the status of the data transfer.

- CPU Utilization Optimization

The CPU does not handle the bulk data transfer.

Instead, the CPU performs only control operations such as:

• Initializing the NVMe controller.
• Configuring the DMA controller.
• Starting the storage operation.
• Handling the completion interrupt.
• Processing the transferred data after completion.

The actual bulk data movement is performed by the DMA controller.

- Application of Computer Architecture Concepts

The proposed system applies the following concepts:

• PCIe – Provides high-speed communication between the system and NVMe controller.
• NVMe – Provides an efficient interface for high-speed SSD storage.
• DMA – Transfers bulk data directly between the storage device and main memory.
• Interrupts – Notify the CPU when the transfer is completed.
• Main Memory – Stores the data transferred from the NVMe storage device.
• CPU – Performs control and processing operations instead of handling every byte of data.

- Constraint Satisfaction

Throughput greater than 3 GB/s:

NVMe over PCIe is selected as the high-speed storage interface to support the required high data-transfer rate.

NVMe over PCIe:

The NVMe controller communicates with the storage device through PCIe as required.

CPU should not handle bulk data transfer:

DMA performs the bulk data transfer directly between the storage device and main memory.

DMA with interrupt notification:

DMA generates an interrupt after completing the transfer, allowing the CPU to respond without continuously monitoring the transfer.

- Justification

NVMe over PCIe is selected because the system requires high-speed storage communication with throughput greater than 3 GB/s. DMA is selected because the CPU should not handle the bulk data transfer.

Using DMA allows the CPU to perform other tasks while data is transferred directly between the storage device and main memory. An interrupt is generated after the transfer is completed, allowing the CPU to efficiently respond to the completion event.

Therefore, the combination of NVMe, PCIe, DMA, main memory, and interrupt notification provides an efficient high-speed I/O subsystem that satisfies all the given constraints.

- Conclusion

The proposed I/O subsystem uses NVMe over PCIe for high-speed storage communication and DMA for direct bulk data transfer. The CPU is used mainly for configuration and control operations rather than transferring the data itself.

After the DMA transfer is completed, an interrupt notification is generated to inform the CPU. This architecture provides high-speed storage communication, minimizes CPU involvement, and satisfies the requirement for throughput greater than 3 GB/s.

Q3. Construct a multi-device communication architecture with the following constraints:

• 6 peripherals using USB.
• 2 high-speed devices using Thunderbolt.
• Avoid interrupt starvation.
• Use hardware and software interrupts.
• Ensure fair bus arbitration.

ANSWER:

- Problem Understanding

The problem is to design a multi-device communication architecture that supports six USB peripherals and two high-speed Thunderbolt devices.

Since multiple devices may request CPU attention at the same time, the system must avoid interrupt starvation. The architecture must support both hardware and software interrupts. It must also provide fair access to the communication bus so that one device does not continuously occupy the bus.

Therefore, an interrupt controller and a fair bus arbitration mechanism are required to manage communication between the CPU and multiple devices.

- Constraint Analysis

The major constraints of the system are:

- Six peripherals must communicate using USB.

- Two high-speed devices must communicate using Thunderbolt.

- Interrupt starvation must be avoided.

- Both hardware and software interrupts must be supported.

- Bus access must be fairly distributed among devices.

The main design priorities are reliable interrupt handling, fair bus access, and efficient communication between multiple devices.

- Proposed Architecture

The proposed architecture is:

+----------------+

|      CPU       |

+-------+--------+

|

|

+-------v--------+

|    Interrupt   |

|    Controller  |

+-------+--------+

|

+---------------+---------------+

|                               |

Hardware Interrupts              Software Interrupts

|                               |

v                               v

+-------------------+             +----------------+

| USB Controller    |             | Operating      |

|                   |             | System /      |

+---------+---------+             | Background     |

|                       | Tasks          |

+---------+---------+             +----------------+

|    |    |    |    |    |

v    v    v    v    v    v

USB1 USB2 USB3 USB4 USB5 USB6

+-------------------+

| Thunderbolt       |

| Controller        |

+---------+---------+

|

+------+------+

|             |

v             v

Thunderbolt 1  Thunderbolt 2

|

v

+--------------+

| Bus Arbiter  |

|              |

| Round-Robin  |

+--------------+

4. Explanation of the Architecture

CPU

The CPU performs the main processing operations and responds to hardware and software interrupts. It does not continuously control every device because the interrupt controller and device controllers manage communication.

Interrupt Controller

The interrupt controller receives interrupt requests from the USB and Thunderbolt controllers. It determines which interrupt should be serviced and sends the appropriate interrupt request to the CPU.

USB Controller

The USB controller manages the six USB peripherals. The six devices are connected through the USB controller or USB hub.

The devices can generate hardware interrupts when important events such as data availability or transfer completion occur.

Thunderbolt Controller

The Thunderbolt controller manages the two high-speed Thunderbolt devices. Thunderbolt is used for high-speed communication as specified in the question.

Bus Arbiter

The bus arbiter controls access to the shared communication bus. It ensures that multiple devices get a fair opportunity to use the bus.

- Hardware Interrupts

Hardware interrupts are generated by physical devices when they require immediate attention from the CPU.

In this architecture, USB and Thunderbolt controllers can generate hardware interrupts.

The basic process is:

- A device generates an interrupt.

- The interrupt is received by the interrupt controller.

- The interrupt controller determines the interrupt source.

- The CPU receives the interrupt.

- The CPU executes the corresponding Interrupt Service Routine.

- The device interrupt is serviced.

- The CPU continues its previous operation.

- Software Interrupts

Software interrupts are generated by software rather than directly by hardware devices.

They can be used for operating-system services, background operations, and deferred processing.

For example, after a hardware interrupt is received and the immediate device operation is completed, additional processing can be performed using software-based mechanisms.

This prevents lengthy operations from being performed inside the hardware interrupt service routine.

- Avoiding Interrupt Starvation

Interrupt starvation occurs when a low-priority device does not get sufficient CPU service because higher-priority interrupts continuously occupy the processor.

To avoid interrupt starvation, the system can use controlled priority scheduling and fair interrupt servicing.

The interrupt controller should give high-priority interrupts quick service while ensuring that lower-priority interrupts are also serviced within a reasonable time.

Interrupt priorities should therefore be carefully assigned so that no device is permanently blocked.

- Fair Bus Arbitration

Fair bus arbitration is required because several USB and Thunderbolt devices may request access to the communication bus.

A round-robin arbitration technique can be used.

Example:

USB 1 → USB 2 → USB 3 → USB 4 → USB 5 → USB 6 → Thunderbolt 1 → Thunderbolt 2 → Repeat

Each device receives an opportunity to use the bus.

If a device does not require the bus, the arbiter moves to the next device. This prevents a single device from continuously occupying the bus.

- Working of the Complete System

- The USB and Thunderbolt devices communicate through their respective controllers.

- When a device requires CPU attention, it generates a hardware interrupt.

- The interrupt controller receives and manages the interrupt.

- The CPU executes the appropriate interrupt service routine.

- Non-critical processing can be handled using software interrupts or background tasks.

- When multiple devices request bus access, the bus arbiter determines which device gets access.

- Round-robin arbitration provides fair access to the bus.

- The process continues continuously as devices communicate with the system.

- Application of Computer Architecture Concepts

The proposed architecture applies the following concepts:

• USB – Used to connect six peripherals.
• Thunderbolt – Used to connect two high-speed devices.
• Hardware Interrupts – Used for immediate notification from devices.
• Software Interrupts – Used for operating-system and background operations.
• Interrupt Controller – Used to manage multiple interrupt requests.
• Bus Arbitration – Used to control access to the shared bus.
• Round-Robin Arbitration – Used to provide fair access to all devices.

- Constraint Satisfaction

Six USB peripherals:

A USB controller or USB hub is used to connect and manage six USB peripherals.

Two high-speed Thunderbolt devices:

A Thunderbolt controller is used to manage the two high-speed devices.

Avoid interrupt starvation:

Controlled interrupt priority and fair servicing are used to ensure that lower-priority devices are not permanently blocked.

Hardware and software interrupts:

Hardware interrupts are used for device events, while software interrupts are used for operating-system and background tasks.

Fair bus arbitration:

Round-robin arbitration provides each device with an opportunity to access the bus.

- Justification

The proposed architecture uses separate USB and Thunderbolt controllers to manage the connected devices efficiently. An interrupt controller manages hardware interrupt requests and prevents uncontrolled interrupt handling.

Software interrupts are used for background and operating-system operations, reducing the workload inside hardware interrupt service routines.

A round-robin bus arbitration mechanism provides fair access to the bus by allowing each device to receive an opportunity to communicate. This reduces the possibility of one device continuously occupying the bus.

Therefore, the proposed architecture satisfies all the constraints by combining USB, Thunderbolt, hardware interrupts, software interrupts, interrupt management, and fair bus arbitration.

- Conclusion

The proposed multi-device communication architecture supports six USB peripherals and two high-speed Thunderbolt devices. Hardware interrupts are used for immediate device events, while software interrupts are used for background and operating-system tasks.

Interrupt starvation is reduced through controlled interrupt servicing, and fair bus access is achieved using round-robin arbitration.

Q4. Design a data acquisition system with the following constraints:

• Continuous data stream at 500 MB/s.
• Minimal CPU intervention required.
• Must compare DMA transfer vs interrupt-driven I/O.
• Use memory-mapped I/O for device registers.

ANSWER:

- Problem Understanding

The problem is to design a high-speed data acquisition system that continuously receives data at a rate of 500 MB/s.

Since the data stream is continuous and has a high transfer rate, the CPU should not be involved in every individual data transfer. Therefore, a Direct Memory Access (DMA) controller is used to transfer data directly from the data acquisition device to main memory.

The system must also use Memory-Mapped I/O for accessing the device registers. DMA transfer and interrupt-driven I/O must be compared to determine the most suitable technique for this high-speed application.

- Constraint Analysis

The major constraints of the system are:

- The system must support a continuous data stream of 500 MB/s.

- CPU intervention must be minimized.

- DMA transfer must be compared with interrupt-driven I/O.

- Memory-Mapped I/O must be used for device registers.

- The system should provide efficient and continuous data transfer.

The main requirement is to transfer a large amount of continuous data while keeping CPU utilization low. Therefore, DMA is more suitable than interrupt-driven I/O for the actual data transfer.

- Proposed Architecture

The proposed architecture is:

+-------------------------+

|   Data Acquisition      |

|        Device           |

+------------+------------+

|

| 500 MB/s

|

v

+-------------------------+

|     DMA Controller      |

+------------+------------+

|

| Direct Data Transfer

|

v

+-------------------------+

|      Main Memory        |

|     Data Buffer         |

+------------+------------+

|

v

+-------------------------+

|    Data Processing      |

|         Unit            |

+-------------------------+

+----------------+

|      CPU       |

+-------+--------+

|

| Memory-Mapped I/O

|

v

+----------------+

| Device Control |

|   Registers    |

+----------------+

DMA Controller

|

| Transfer Complete

v

+----------------+

|    Interrupt   |

|    Controller  |

+-------+--------+

|

v

CPU

4. Explanation of the Architecture

Data Acquisition Device

The data acquisition device continuously produces data at a rate of 500 MB/s. The data must be transferred to main memory efficiently without requiring continuous CPU intervention.

DMA Controller

The DMA controller performs the bulk data transfer. It transfers data directly from the data acquisition device to main memory.

This allows the CPU to perform other tasks while the data transfer is in progress.

Main Memory

Main memory is used as a buffer for storing the incoming data. The DMA controller writes the acquired data directly into the allocated memory area.

Data Processing Unit

After the data is stored in memory, the processor or data processing unit can process the acquired data.

CPU

The CPU is mainly responsible for configuring the data acquisition device and DMA controller. It does not handle every individual data transfer.

Memory-Mapped I/O

Memory-Mapped I/O is used to access the control and status registers of the data acquisition device. The device registers are assigned addresses within the processor's memory address space.

- Working of the Proposed System

- The CPU initializes the data acquisition device.

- The CPU configures the DMA controller with the source, destination, and transfer size.

- The data acquisition device starts producing data at 500 MB/s.

- The DMA controller transfers the data directly from the device to main memory.

- The CPU continues performing other operations while DMA performs the transfer.

- The data is stored in a memory buffer.

- After completing a data block, the DMA controller generates an interrupt.

- The CPU receives the interrupt notification.

- The CPU processes the completed data block.

- The DMA controller continues with the next data transfer.

- Memory-Mapped I/O

Memory-Mapped I/O is used for accessing the registers of the data acquisition device.

The device registers are assigned specific memory addresses. The CPU can access these registers using normal memory read and write operations.

For example, the CPU can use Memory-Mapped I/O to:

• Configure the data acquisition device.
• Start or stop data acquisition.
• Read device status.
• Configure transfer parameters.
• Check error conditions.

Memory-Mapped I/O is used for device control and configuration, while DMA is used for high-speed bulk data transfer.

- Comparison of DMA Transfer and Interrupt-Driven I/O

DMA Transfer:

• Requires minimal CPU intervention.
• Transfers data directly between the device and memory.
• Suitable for high-speed continuous data streams.
• Reduces CPU overhead.
• Provides efficient large-block data transfer.
• Suitable for the required 500 MB/s data rate.

Interrupt-Driven I/O:

• The device generates an interrupt when it requires CPU attention.
• The CPU executes an Interrupt Service Routine.
• The CPU is involved in managing the data transfer.
• Frequent interrupts can increase CPU overhead.
• Less suitable for continuous high-speed data streams.
• Suitable for smaller or moderate data transfers.

- DMA vs Interrupt-Driven I/O

| Feature | DMA Transfer | Interrupt-Driven I/O |
| --- | --- | --- |
| CPU involvement | Very low | High |
| Data transfer | Device directly to memory | CPU manages transfer |
| CPU overhead | Low | High |
| High-speed data | Highly suitable | Less suitable |
| Continuous 500 MB/s stream | Suitable | Not preferred |
| Large data blocks | Suitable | Less efficient |
| Performance | High | Lower for high data rates |
| Selected for this system | Yes | No |

- Selection of DMA

DMA is selected for this system because the data acquisition device continuously generates data at 500 MB/s.

If interrupt-driven I/O were used for every data transfer, the CPU would receive a large number of interrupts and would have to perform more transfer operations. This would increase CPU utilization and reduce the efficiency of the system.

With DMA, the CPU only configures the transfer and receives an interrupt after a block of data has been transferred. Therefore, CPU intervention is minimized.

- CPU Intervention

The CPU performs only important control operations:

• Initializes the data acquisition device.
• Configures the DMA controller.
• Specifies the memory buffer.
• Starts the data acquisition process.
• Receives the DMA completion interrupt.
• Processes the acquired data.

The CPU does not copy every data item from the device to memory. This significantly reduces CPU workload.

- Application of Computer Architecture Concepts

The proposed system applies the following concepts:

• DMA – Used for direct transfer of data between the device and memory.
• Memory-Mapped I/O – Used for accessing device control and status registers.
• Interrupts – Used to notify the CPU after completion of a DMA transfer.
• Main Memory – Used as a buffer for the continuous data stream.
• CPU – Used mainly for configuration, control, and data processing.
• Data Acquisition Device – Generates the continuous 500 MB/s data stream.

- Constraint Satisfaction

Continuous data stream of 500 MB/s:

DMA provides efficient high-speed transfer between the data acquisition device and main memory.

Minimal CPU intervention:

The DMA controller performs the bulk data transfer, so the CPU does not handle every data item.

Comparison of DMA and interrupt-driven I/O:

DMA is more suitable for the high-speed continuous data stream because it has lower CPU overhead than interrupt-driven I/O.

Memory-Mapped I/O:

Memory-Mapped I/O is used to access the device registers for configuration and control.

- Justification

DMA is the most suitable transfer mechanism for this system because the data stream is continuous and has a high rate of 500 MB/s. Using interrupt-driven I/O for every transfer would cause excessive CPU involvement and increase CPU overhead.

Memory-Mapped I/O provides efficient access to the data acquisition device's registers. DMA handles the actual bulk data movement, while interrupts are used only to notify the CPU about important events such as transfer completion.

Therefore, the combination of Memory-Mapped I/O, DMA, main memory buffering, and interrupt notification provides an efficient solution for the given data acquisition requirements.

- Conclusion

The proposed data acquisition system uses Memory-Mapped I/O for device registers and DMA for high-speed data transfer.

The data acquisition device continuously produces data at 500 MB/s, and the DMA controller transfers this data directly to main memory. The CPU is involved mainly in configuration, control, and processing rather than bulk data movement.

Compared with interrupt-driven I/O, DMA provides lower CPU overhead and is more suitable for continuous high-speed data acquisition.

Q5. Construct an interrupt handling mechanism with the following constraints:

• Support nested interrupts.
• Maximum interrupt latency must be less than 10 µs.
• Use vectored interrupts for high-priority devices.
• Use software interrupts for background tasks.

ANSWER:

- Problem Understanding

The problem is to design an interrupt handling mechanism for a system that must respond to important events within a maximum interrupt latency of less than 10 µs.

The system must support nested interrupts so that a high-priority interrupt can interrupt the execution of a lower-priority interrupt service routine. Vectored interrupts must be used for high-priority devices to provide fast identification of the interrupt source.

Software interrupts are used for background tasks that do not require immediate hardware response. The overall mechanism should provide fast and efficient interrupt handling while satisfying the latency requirement.

- Constraint Analysis

The major constraints of the system are:

- The system must support nested interrupts.

- Maximum interrupt latency must be less than 10 µs.

- High-priority devices must use vectored interrupts.

- Background tasks must use software interrupts.

- Interrupt Service Routines should be short and efficient.

- Interrupt priorities must be managed properly.

The most important constraint is the maximum interrupt latency of less than 10 µs. Therefore, high-priority interrupts must be identified and serviced quickly.

- Proposed Architecture

The proposed interrupt handling architecture is:

+------------------+

|     Devices      |

+--------+---------+

|

| Interrupt Requests

v

+----------------------+

| Interrupt Controller |

+----------+-----------+

|

Priority Checking

|

+--------------+--------------+

|                             |

v                             v

High-Priority IRQ              Low-Priority IRQ

|                             |

v                             v

+------------------+          +------------------+

| Vectored         |          | Normal Interrupt |

| Interrupt        |          | Handling         |

+--------+---------+          +--------+---------+

|                             |

v                             |

+-----------+                       |

| High       |                       |

| Priority   |                       |

| ISR        |                       |

+-----+------+                       |

|                              |

| Nested Interrupt            |

|                              |

v                              |

+-----------+                        |

| Save CPU  |                        |

| Context   |                        |

+-----+-----+                        |

|                              |

v                              |

+-----------+                        |

| Service   |                        |

| High IRQ  |                        |

+-----+-----+                        |

|                              |

v                              |

+-----------+                        |

| Restore   |                        |

| Context   |                        |

+-----+-----+                        |

|                              |

+--------------+---------------+

|

v

+-------+

|  CPU  |

+---+---+

|

v

Software Interrupts

|

v

Background Tasks

4. Explanation of the Architecture

Interrupting Devices

Devices generate interrupt requests when they require the attention of the CPU. These requests are sent to the interrupt controller.

Interrupt Controller

The interrupt controller receives interrupt requests from different devices. It determines the priority of the interrupts and forwards the appropriate interrupt request to the CPU.

High-Priority Devices

High-priority devices use vectored interrupts. Each high-priority device is assigned a specific interrupt vector.

When the interrupt occurs, the CPU can directly locate the corresponding Interrupt Service Routine instead of searching for the interrupt source.

High-Priority ISR

The high-priority Interrupt Service Routine performs only the essential operations required to handle the interrupt. Keeping the ISR short helps to reduce interrupt latency.

CPU Context

When a high-priority interrupt occurs during the execution of another ISR, the current CPU context is saved. This allows the CPU to return to the previous task after the high-priority interrupt has been serviced.

Software Interrupts

Software interrupts are used for background tasks and operations that do not require immediate hardware response. This separates non-critical processing from time-critical hardware interrupt handling.

- Nested Interrupt Handling

Nested interrupts allow a higher-priority interrupt to interrupt the execution of a lower-priority interrupt.

The sequence is:

- A low-priority interrupt is being serviced.

- A high-priority interrupt occurs.

- The interrupt controller identifies the higher priority.

- The current CPU context is saved.

- The CPU accepts the high-priority interrupt.

- The CPU uses the interrupt vector to locate the high-priority ISR.

- The high-priority ISR is executed.

- The high-priority interrupt is completed.

- The saved CPU context is restored.

- The previous low-priority ISR continues its execution.

- Maintaining Interrupt Latency Below 10 µs

The following techniques are used to maintain the maximum interrupt latency below 10 µs:

• Use vectored interrupts for high-priority devices.
• Keep high-priority ISRs short.
• Avoid lengthy processing inside the ISR.
• Assign appropriate priorities to interrupts.
• Use efficient context saving and restoration.
• Move non-critical operations to software interrupts or background tasks.
• Avoid unnecessary interrupt disabling.

Vectored interrupts reduce the time required to identify the source of a high-priority interrupt, while short ISRs reduce the total time required to service the interrupt.

- Vectored Interrupts for High-Priority Devices

Vectored interrupts are used for devices that require immediate attention.

Each high-priority device is associated with a specific interrupt vector. When the device generates an interrupt, the CPU receives the corresponding vector and directly transfers control to the required ISR.

This reduces interrupt identification time and helps the system achieve the required latency of less than 10 µs.

- Software Interrupts for Background Tasks

Software interrupts are used for tasks that do not require immediate hardware response.

Examples include:

• Background processing.
• Operating-system services.
• Deferred processing.
• Non-critical data processing.
• System-level software operations.

Using software interrupts for these tasks prevents unnecessary processing from being performed inside high-priority hardware ISRs.

- Working of the Complete System

- Devices generate interrupt requests.

- The interrupt controller receives the requests.

- The controller determines the priority of each interrupt.

- A high-priority device generates a vectored interrupt.

- The CPU directly executes the corresponding high-priority ISR.

- If another higher-priority interrupt occurs, nested interrupt handling is performed.

- The current CPU context is saved.

- The higher-priority ISR is executed.

- After completion, the previous CPU context is restored.

- Background operations are handled using software interrupts.

- The CPU continues normal program execution.

- Application of Computer Architecture Concepts

The proposed mechanism applies the following concepts:

• Nested Interrupts – Allow higher-priority interrupts to interrupt lower-priority interrupt processing.
• Vectored Interrupts – Provide fast identification of high-priority interrupt sources.
• Interrupt Controller – Manages multiple interrupt requests and priorities.
• Interrupt Service Routine – Performs the required interrupt handling.
• Context Saving – Preserves the state of the interrupted task.
• Software Interrupts – Used for background and non-critical tasks.
• Interrupt Priority – Ensures that important devices receive immediate service.

- Constraint Satisfaction

Support nested interrupts:

The architecture saves the current CPU context and allows a higher-priority interrupt to interrupt a lower-priority ISR.

Maximum latency below 10 µs:

Vectored interrupts, short ISRs, proper interrupt priorities, and efficient context switching are used to minimize interrupt latency.

Vectored interrupts for high-priority devices:

High-priority devices are assigned specific interrupt vectors so that the CPU can directly locate their ISRs.

Software interrupts for background tasks:

Non-critical and background operations are handled using software interrupts rather than high-priority hardware interrupts.

- Justification

Nested interrupt handling is selected because high-priority events must be serviced immediately even when the CPU is already handling a lower-priority interrupt.

Vectored interrupts are used for high-priority devices because they reduce the time required to identify the interrupt source. Short ISRs further reduce interrupt processing time and help meet the latency requirement of less than 10 µs.

Software interrupts are used for background tasks because these tasks do not require immediate hardware response. Moving such operations away from high-priority ISRs prevents unnecessary delays.

Therefore, the combination of nested interrupts, vectored interrupts, short ISRs, priority-based interrupt handling, and software interrupts provides an efficient interrupt handling mechanism that satisfies the given constraints.

- Conclusion

The proposed interrupt handling mechanism supports nested interrupts and uses vectored interrupts for high-priority devices. The use of short ISRs, interrupt prioritization, and efficient context handling helps maintain the maximum interrupt latency below 10 µs.

Software interrupts are used for background tasks and non-critical operations so that they do not interfere with time-critical hardware interrupt handling.