SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN) |
| Weightage: | 30% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | N.Nesarajan | Reg.No.: | 192511386 |
| Department: | CSE | Date: | 31-08-2026 |

ANNEXURE A

Pseudocode Writing Task (Algorithm Design)

- Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

- Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

- Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

- Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

- Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Code of Conduct:

I N.Nesarajan/192511386  (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (20%) | Algorithm Design (30%) | Use of Control Structures (20%) | Pseudocode Structure (10%) | Completeness (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |

ANSWERS

QUESTION 1: DMA-Based Data Transfer

Problem Statement

Design a DMA-based data transfer algorithm to transfer data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

Algorithm

- Initialize the DMA controller.

- Set the source as the I/O device.

- Set the destination as the required memory address.

- Specify the amount of data to be transferred.

- Start the DMA transfer.

- Allow the CPU to perform other tasks during the transfer.

- After completion, DMA generates an interrupt.

- CPU handles the interrupt and checks the transfer status.

- If successful, acknowledge the interrupt; otherwise, handle the error.

- Disable the DMA channel after completion.

Pseudocode

ALGORITHM DMA_Data_Transfer

BEGIN

Initialize DMA Controller

Source ← I/O Device

Destination ← Memory Address

Transfer_Size ← Number of Bytes

Configure DMA(Source, Destination, Transfer_Size)

Enable DMA Channel

Start DMA Transfer

WHILE DMA Transfer is NOT Complete DO

CPU performs other tasks

END WHILE

IF DMA Interrupt is Received THEN

Check DMA Status

IF Transfer Status = SUCCESS THEN

Acknowledge Interrupt

Display "DMA Transfer Completed"

ELSE

Handle DMA Transfer Error

END IF

END IF

Disable DMA Channel

END

Explanation

DMA allows data to be transferred directly between an I/O device and memory without requiring the CPU to manage every data byte. The CPU only initializes the DMA controller and can perform other tasks while the transfer is in progress. Once the transfer is completed, the DMA controller sends an interrupt to the CPU. This reduces CPU involvement and improves system performance.

QUESTION 2: Prioritized Vectored Interrupt Handling

Problem Statement

Develop an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches the appropriate service routines.

Algorithm

- Initialize the interrupt controller.

- Maintain a list of connected devices and their priorities.

- Continuously monitor interrupt requests.

- Identify the device generating the interrupt.

- Obtain its interrupt vector.

- If multiple interrupts occur, select the highest-priority interrupt.

- Save the current CPU state.

- Execute the corresponding Interrupt Service Routine (ISR).

- Acknowledge the interrupt.

- Restore the CPU state and continue normal execution.

Pseudocode

ALGORITHM Priority_Vectored_Interrupt_Handler

BEGIN

Initialize Interrupt Controller

Create Priority List for Devices

WHILE System is Running DO

IF Interrupt Request is Received THEN

Identify Interrupting Device

Read Interrupt Vector

IF Multiple Interrupts are Pending THEN

Select Highest Priority Interrupt

END IF

Save CPU State

Disable Lower Priority Interrupts

ISR ← Get Service Routine(Interrupt Vector)

Execute ISR

Acknowledge Interrupt

Restore CPU State

Enable Interrupts

END IF

END WHILE

END

Explanation

A vectored interrupt provides information that identifies the service routine required for a particular device. When several devices request interrupts at the same time, the interrupt controller determines their priority and services the highest-priority request first. The CPU saves its current state, executes the appropriate ISR, acknowledges the interrupt, and then resumes the interrupted task.

QUESTION 3: Dynamic Selection of Memory-Mapped I/O and I/O-Mapped I/O

Problem Statement

Write pseudocode to dynamically select between memory-mapped I/O and I/O-mapped I/O based on latency and bandwidth requirements.

Algorithm

- Read the requirements of the I/O device.

- Determine the required latency.

- Determine the required bandwidth.

- If both latency and bandwidth requirements are high, select memory-mapped I/O.

- If requirements are low, select I/O-mapped I/O.

- For intermediate requirements, compare system resources and performance.

- Configure the selected I/O technique.

- Return the selected method.

Pseudocode

ALGORITHM Dynamic_IO_Mapping

BEGIN

Read Device Requirements

Latency ← Get Latency Requirement

Bandwidth ← Get Bandwidth Requirement

IF Latency = HIGH AND Bandwidth = HIGH THEN

Select IO_Method ← Memory_Mapped_IO

ELSE IF Latency = LOW AND Bandwidth = LOW THEN

Select IO_Method ← IO_Mapped_IO

ELSE

Compare System Performance

IF Memory Availability is HIGH THEN

Select IO_Method ← Memory_Mapped_IO

ELSE

Select IO_Method ← IO_Mapped_IO

END IF

END IF

Configure IO_Method

Display Selected IO_Method

END

Explanation

Memory-mapped I/O and I/O-mapped I/O are two techniques used for communication between the CPU and I/O devices. The algorithm selects the appropriate technique according to latency and bandwidth requirements. Memory-mapped I/O can provide efficient access using normal memory instructions, while I/O-mapped I/O uses a separate I/O address space. Dynamic selection helps the system choose a suitable method for different device requirements.

QUESTION 4: Bus Arbitration for PCIe, USB and Thunderbolt

Problem Statement

Design a bus arbitration algorithm that manages multiple devices communicating through PCIe, USB, and Thunderbolt interfaces.

Algorithm

- Initialize PCIe, USB, and Thunderbolt interfaces.

- Monitor requests from all connected devices.

- Add active requests to a request queue.

- Assign priority to each request.

- Select the highest-priority request.

- Allocate the required interface resources.

- Allow the selected device to transfer data.

- After completion, release the resources.

- Process the next request.

- Keep the bus/interface idle when no request is present.

Pseudocode

ALGORITHM Bus_Arbitration

BEGIN

Initialize PCIe Interface

Initialize USB Interface

Initialize Thunderbolt Interface

Create Request Queue

WHILE System is Running DO

Check PCIe Requests

Check USB Requests

Check Thunderbolt Requests

Add Active Requests to Queue

IF Request Queue is NOT Empty THEN

Calculate Priority for Each Request

Selected_Request ← Highest Priority Request

IF Selected_Request Interface = PCIe THEN

Allocate PCIe Resources

ELSE IF Selected_Request Interface = USB THEN

Allocate USB Resources

ELSE IF Selected_Request Interface = Thunderbolt THEN

Allocate Thunderbolt Resources

END IF

Allow Data Transfer

Update Interface Status

Release Allocated Resources

Remove Completed Request from Queue

ELSE

Keep Interfaces Available

END IF

END WHILE

END

Explanation

Bus arbitration determines which device gets access to shared communication resources when multiple devices request data transfer. The algorithm checks PCIe, USB, and Thunderbolt requests and places them into a queue. Priority is then calculated and the highest-priority request is selected. After the transfer is completed, resources are released so that another device can communicate.

QUESTION 5: Hybrid I/O Communication Mechanism

Problem Statement

Write pseudocode for a hybrid I/O mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Algorithm

- Initialize all I/O devices.

- Determine the speed of each device.

- For low-speed devices, use polling.

- Continuously check the device status.

- When data is available, read and store it in memory.

- For high-speed devices, initialize the DMA controller.

- Configure source, destination, and transfer size.

- Start the DMA transfer.

- Allow the CPU to perform other tasks.

- DMA generates an interrupt after completion.

- CPU handles the interrupt and checks the transfer status.

- Disable DMA after the transfer.

Pseudocode

ALGORITHM Hybrid_IO_Communication

BEGIN

Initialize All I/O Devices

FOR Each Device DO

Device_Speed ← Determine Device Speed

IF Device_Speed = LOW THEN

Enable Polling

WHILE Data Transfer is NOT Complete DO

Check Device Status

IF Data is Available THEN

Read Data from Device

Store Data in Memory

END IF

END WHILE

Disable Polling

ELSE IF Device_Speed = HIGH THEN

Initialize DMA Controller

Source ← I/O Device

Destination ← Memory

Transfer_Size ← Required Data Size

Configure DMA(Source, Destination, Transfer_Size)

Enable DMA Interrupt

Start DMA Transfer

WHILE DMA Transfer is Active DO

CPU performs other tasks

END WHILE

IF DMA Interrupt is Received THEN

Check DMA Status

IF Transfer Status = SUCCESS THEN

Acknowledge Interrupt

ELSE

Handle DMA Error

END IF

END IF

Disable DMA

END IF

END FOR

Display "I/O Communication Completed"

END

Explanation

The hybrid I/O mechanism combines polling and DMA according to the speed of the device. Polling is suitable for low-speed devices because the amount of data and number of requests are relatively small. High-speed devices use DMA with interrupts because DMA can transfer large amounts of data while reducing CPU involvement. The interrupt informs the CPU when the DMA operation has finished. This provides efficient I/O communication and better CPU utilization.

Conclusion

The five algorithms demonstrate important I/O communication mechanisms used in computer architecture. DMA reduces CPU involvement during large data transfers, while vectored interrupts provide efficient handling of multiple device requests. Memory-mapped I/O and I/O-mapped I/O provide different methods for accessing I/O devices. Bus arbitration manages communication requests from different interfaces such as PCIe, USB, and Thunderbolt. Finally, the hybrid I/O mechanism combines polling and DMA with interrupts to provide efficient communication for both low-speed and high-speed devices. These approaches improve system performance, resource utilization, and efficient CPU operation.